## PS-01: Device-Free WiFi Sensing & Movement Analytics

Domain: Ambient Intelligence, Smart Buildings, & Privacy-Preserving Sensing

Primary Goal: Detect human presence, count occupants, and classify activities/falls using off-the-shelf WiFi Channel State Information (CSI) without relying on cameras, microphones, or wearables.

### 1. Business Context & Problem Description

Traditional occupancy and activity monitoring rely heavily on optical cameras or wearable sensors, raising severe privacy concerns, high hardware costs, and maintenance hurdles. WiFi Channel State Information (CSI) measures the amplitude and phase disturbances caused by human movement in RF multipath propagation fields. Leveraging standard WiFi signals enables non-line-of-sight (NLOS) sensing through walls while maintaining complete privacy.

### 2. Core Objectives

- Ingest and parse raw CSI/RSSI telemetry from standard WiFi hardware (e.g., ESP32 with CSI firmware or Raspberry Pi with Nexmon patch).

- Perform signal conditioning to eliminate environmental drift, high-frequency noise, and phase distortion.

- Train lightweight edge machine learning models to classify ambient state variations (e.g., empty room, static person, walking, falling, occupant count).

- Deploy real-time inference on low-power edge nodes (TinyML) without cloud dependencies.

### 3. Key Technical Challenges

- Low Signal-to-Noise Ratio (SNR): Multipath fading, environmental static noise, and wall attenuation obscure subtle human motion signals.

- Hardware & Phase Instability: Carrier frequency offset (CFO) and sampling time offset (STO) in consumer WiFi chipsets cause non-stationary phase noise.

- Edge Resource Constraints: Executing signal preprocessing and Deep Learning inference within the tight memory and CPU constraints of ESP32/Raspberry Pi microcontrollers.

### 4. Technical Requirements & Deliverables

- CSI Data Ingestion & Preprocessing Pipeline: Real-time signal filtering framework incorporating Hampel filters, Principal Component Analysis (PCA), and wavelet denoising.

- Edge Deep Learning Model: Lightweight classification architecture (e.g., 1D CNN-LSTM or MobileNet-1D) trained on calibrated CSI feature maps.

- TinyML Runtime Deployment: Quantized model artifact (TFLite Micro/ONNX) running locally on ESP32/Raspberry Pi edge hardware.

### 5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Activity Classification F1-Score | ≥90% accuracy across empty, sitting, walking, and fall events. |
| Fall Detection Recall | ≥98% recall to prevent undetected fall emergencies. |
| Inference Latency | <100 ms processing time per window on edge hardware. |
| Occupancy Count Error | Mean Absolute Error (MAE) ≤1 person in adjacent-room setups. |

PS-02: Secure Examination Paper Storage & Transit System

Domain: Education Governance, IoT Security, & Supply Chain Safeguarding

Primary Goal: Prevent question paper leaks during storage and transit using smart sensor-equipped containers, dynamic electronic locking, real-time telemetry, and immutable audit logs.

1. Business Context & Problem Description

High-stakes academic and recruitment examinations frequently face compromises due to physical tampering during storage and transit. Manual lock-and-key protocols and offline surveillance fail to provide proactive breach visibility. An IoT-based secure ecosystem provides real-time tracking, environmental verification, tamper detection, and immediate security escalation.

2. Core Objectives

- Design tamper-proof smart storage containers equipped with electronic locks, GPS tracking, and environmental sensors.

- Enforce multi-factor authentication (MFA) and geofenced access policies for authorized personnel.

- Stream continuous container health (location, lid status, vibration/tilt, ambient conditions) to a central cloud dashboard.

- Maintain an immutable audit log of every access event to ensure complete chain-of-custody accountability.

3. Key Technical Challenges

- Intermittent Connectivity: Maintaining telemetry logging and security integrity during transportation across rural or low-network zones.

- Physical Tamper Evasion: Detecting sophisticated intrusion attempts (e.g., side-casing breach, lock bypass, sensor spoofing).

- Power Management: Ensuring continuous operation across multi-day transit routes using low-power IoT architectures.

4. Technical Requirements & Deliverables

- Smart Enclosure Telematics Module: Hardware node integrating cellular/GPS, micro-switches, optical light sensors, accelerometers, and solenoid locks.

- Immutable Audit Engine: Cryptographically verified event ledger logging access requests, location stamps, and sensor triggers.

- Central Command Dashboard: Web/mobile monitoring platform with automated real-time alert dispatching (SMS, Email, Push) upon breach detection.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Tamper Detection Latency | Instantaneous trigger and alert transmission (<2 seconds). |
| Audit Trail Completeness | 100% verifiable logging of all access and sensor events. |
| Geofence Policy Enforcement | 0% unauthorized unlock allowances outside designated examination centers. |

PS-03: IoT Smart Reverse Vending Machine (Bottle Crusher)

Domain: Smart Cities, Environmental Sustainability, & Circular Economy

Primary Goal: Automate plastic bottle detection, reject non-recyclable contaminants, reduce waste volume via mechanical crushing, and motivate citizens through integrated reward programs.

1. Business Context & Problem Description

Public recycling bins suffer from high contamination rates due to non-plastic waste (metal cans, glass, organic trash) mixed with recyclable PET bottles. Furthermore, uncompressed bottles rapidly fill bin capacity, leading to frequent logistics overruns. An automated Reverse Vending Machine (RVM) validates materials, compacts waste, and incentivizes recycling through instant user rewards.

2. Core Objectives

- Distinguish PET plastic bottles from non-plastic materials (metals, glass, cardboard) at the point of insertion.

- Mechanically crush accepted plastic containers to reduce storage volume by up to 70--80%.

- Attribute recycling contributions to individual user accounts and disburse rewards (points, cashback, coupons).

- Stream real-time operational status (bin fill level, crusher motor health, user activity) to a cloud management dashboard.

3. Key Technical Challenges

- Rapid Material Sorting: Accurate multi-sensor verification (inductive, capacitive, optical, or vision-based) within milliseconds.

- Mechanical Reliability: Preventing motor jams, blade binding, and safely handling fluids remaining in bottles.

- Anti-Fraud Mechanisms: Preventing string-tethered bottle re-insertion and duplicate reward claims.

4. Technical Requirements & Deliverables

- Material Verification Assembly: Multi-sensor array and lightweight vision classifier detecting plastic vs. metal/glass.

- Crusher Control System: Industrial actuator/motor controller equipped with auto-reverse anti-jamming logic.

- User Rewards Platform: Interactive touch frontend with mobile backend integration for instant wallet/reward credit.

- IoT Fleet Management Dashboard: Cloud portal monitoring fill levels, maintenance alerts, and aggregate recycling analytics.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Material Sorting Accuracy | ≥98% correct acceptance of plastic PET bottles and rejection of foreign objects. |
| Volume Reduction Ratio | ≥75% reduction in compressed storage space required per bottle. |
| Reward Settlement Speed | <1.5 seconds execution time from crushing completion to reward credit. |

PS-04: AI Road Construction Material & Granulometry Inspection

Domain: Civil Engineering, Construction Quality Assurance, & Computer Vision

Primary Goal: Automatically identify road construction raw materials and estimate aggregate stone chip size distributions in real time using computer vision and deep learning.

1. Business Context & Problem Description

The structural lifespan and load distribution of asphalt roads depend heavily on aggregate gradation (stone chip size mix). Manual sieve analysis and visual site inspections are labor-intensive, slow, and subjective. Deploying a camera-based AI inspection system provides continuous, non-destructive quality control directly on site.

2. Core Objectives

- Classify road construction raw materials (e.g., stone aggregates, sand, gravel, bitumen mixes) from camera imagery.

- Segment individual stone chips and estimate their size categories (e.g., 10 mm, 20 mm, 40 mm) using computer vision.

- Compare measured aggregate distributions against standardized engineering sieve specs.

- Transmit automated quality logs to a centralized engineering cloud portal.

3. Key Technical Challenges

- Particle Overlap & Occlusion: Disambiguating dense, overlapping aggregate piles on conveyor belts or dump beds.

- Environmental Variance: Handling unstable outdoor lighting, shadows, surface moisture, and dust buildup on camera lenses.

- Scale Calibration: Maintaining millimeter-level spatial resolution without manual physical reference markers.

4. Technical Requirements & Deliverables

- Optical Image Acquisition Rig: Enclosed camera and LED illumination setup for consistent field captures.

- Computer Vision Granulometry Pipeline: Instance segmentation model (e.g., Mask R-CNN, YOLO-Seg) combined with ellipse fitting for particle axis sizing.

- QA Analytics Dashboard: Desktop/cloud application generating material classification reports and cumulative pass-through sieve curves.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Material Classification Accuracy | ≥95% accuracy across aggregate types, sand, and bitumen mixes. |
| Size Estimation Error | Mean Absolute Error (MAE) ≤1.5 mm compared to manual sieve benchmarks. |
| Sieve Curve Correlation (R2) | R2≥0.92 agreement with standard laboratory mechanical sieve curves. |

PS-05: Autonomous Public Toilet Maintenance & Hygiene System

Domain: Smart Cities, Facility Management, & IoT Automation

Primary Goal: Continuous hygiene monitoring, automated self-cleaning and disinfection cycles, odor mitigation, and demand-driven janitorial dispatch for public toilets.

1. Business Context & Problem Description

Public sanitation facilities often deteriorate due to static, scheduled cleaning regimes that ignore actual footfall and usage intensity. Unmanaged odors, empty soap/water reservoirs, and unhygienic surfaces lead to poor public health outcomes. An autonomous IoT ecosystem cleans dynamically post-usage and dispatches janitorial staff based on real-time sensory thresholds.

2. Core Objectives

- Monitor facility usage count, air quality (NH3,H2S), water levels, and consumable capacity via dynamic IoT sensors.

- Automate touchless post-use flushing, floor washing, and aerosol disinfection based on occupancy triggers.

- Actuate automated ventilation fans and deodorizer dispensers when gas levels cross safety thresholds.

- Stream telemetry to a cloud dashboard to optimize janitorial shifts according to dynamic demand.

3. Key Technical Challenges

- Harsh Operating Environment: Sensor degradation caused by high humidity, corrosive cleaning chemicals, and splashing.

- False Occupancy Triggers: Preventing automated cleaning routines while a user is still present inside the cubicle.

- Resource Optimization: Balancing aggressive sanitation cycles against water and electricity conservation constraints.

4. Technical Requirements & Deliverables

- Multi-Sensor IoT Array: Water-proof sensor cluster (ToF/PIR occupancy, NH3/VOC gas sensors, ultrasonic level indicators).

- Automated Actuator Unit: Relays controlling solenoid water valves, floor wash spray nozzles, and ventilation fans.

- Smart Facility Dashboard: Web platform and mobile app delivering real-time status alerts and dynamic staff routing schedules.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Odor Reduction Efficiency | ≥80% reduction in volatile ammonia/sulfide levels post-ventilation trigger. |
| Resource Savings | ≥25% reduction in total water/chemical usage compared to fixed timer flushes. |
| Janitorial Response SLA | Maintenance teams dispatched within <5 minutes of consumable depletion or fault alerts. |

PS-06: IoT Electromagnetic Radiation (EMR) Monitoring System

Domain: Occupational Health & Safety, Environmental Sensing, & Industrial IoT

Primary Goal: Continuously measure indoor electromagnetic radiation (EMR), generate real-time spatial heatmaps, compare exposure against regulatory safety limits, and issue exposure mitigation alerts.

1. Business Context & Problem Description

The proliferation of Wi-Fi access points, cellular towers, high-voltage infrastructure, and industrial electronics elevates electromagnetic field (EMF) exposure in workplaces and residences. Periodic manual inspections fail to catch transient spikes. Continuous IoT monitoring provides clear visibility into EMR hotspots and supports compliance with health standards (e.g., ICNIRP/IEEE).

2. Core Objectives

- Measure high-frequency RF and low-frequency ELF field intensity continuously across indoor facilities.

- Aggregate field measurements via IoT gateway nodes and stream spatial data to a central platform.

- Render real-time 2D/3D indoor radiation heatmaps highlighting hotspot zones.

- Issue immediate alerts and actionable safety recommendations when limits are breached.

3. Key Technical Challenges

- Wideband Sensor Calibration: Maintaining high measurement linearity across a broad frequency spectrum (50 Hz to 6 GHz).

- Spatial Interpolation: Generating accurate indoor EMR continuous heatmaps from a finite set of discrete sensor nodes.

- Signal Interference: Filtering out localized electrostatic or temporary non-hazardous transients without missing true safety violations.

4. Technical Requirements & Deliverables

- Multi-Band EMR Sensor Probe: IoT sensing hardware measuring E-field (V/m), H-field (A/m), and power density (W/m2).

- Heatmap Analytics Engine: Cloud processing pipeline using Kriging or Radial Basis Function (RBF) spatial interpolation algorithms.

- Compliance & Alert Portal: Web application displaying live room layouts, safety threshold overlays, and exposure duration tracking.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Measurement Accuracy | ±1.5 dB alignment with calibrated laboratory spectrum analyzers. |
| Breach Alert Latency | Instant notifications pushed within <1 second of threshold breach. |
| Heatmap Interpolation Accuracy | Root Mean Square Error (RMSE) ≤10% across un-sampled spatial coordinates. |

PS-07: AI-Powered Railway Level Crossing Safety System

Domain: Intelligent Transportation Systems, Railway Safety, & Computer Vision

Primary Goal: Detect vehicles, pedestrians, and animals trapped in railway crossing danger zones in real time using edge AI, issuing instant collision-prevention warnings to control rooms.

1. Business Context & Problem Description

Unmanned and congested railway level crossings are high-risk zones for severe accidents caused by stranded vehicles or trespassing pedestrians. Traditional physical barriers cannot detect obstacles trapped inside the gate perimeter after closure. Vision-based edge monitoring automatically flags hazards in real time, preventing derailments and saving lives.

2. Core Objectives

- Continuously monitor the level crossing grid area using optical/thermal vision sensors.

- Detect and classify trapped obstacles (vehicles, humans, livestock, heavy debris) inside the danger zone.

- Generate instant high-priority alerts to station control rooms and incoming train telemetry systems upon hazard detection.

- Log video evidence and event metrics to a central railway analytics portal.

3. Key Technical Challenges

- All-Weather Reliability: Sustaining high target detection accuracy in dense fog, heavy rain, night darkness, and direct sun glare.

- Ultra-Low False Alarm Rate: Preventing false emergency train halts while maintaining zero missed detection tolerance.

- Edge Processing Speed: Executing object detection and tracking algorithms locally with sub-second transmission delays.

4. Technical Requirements & Deliverables

- Ruggedized Edge Processing Unit: Outdoor edge AI hardware (e.g., NVIDIA Jetson / Industrial IPC) running object detection models.

- Deep Learning Vision Engine: Object detection and multi-object tracking pipeline (e.g., YOLOv8 + ByteTRACK) calibrated to the crossing zone.

- IoT Wireless Alert Gateway: Fail-safe, low-latency communications module (LTE/LoRa/GSM) broadcasting to railway control systems.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Obstacle Detection Recall | 100% detection rate for vehicles and pedestrians within the danger zone boundary. |
| False Alarm Rate (FAR) | <0.1% false alarms per 1,000 operational hours. |
| End-to-End Alert Latency | <500 ms from obstacle entry detection to control room alert trigger. |

PS-08: IoT Smart Bridge Structural Health Monitoring (SHM)

Domain: Infrastructure Civil Engineering, Predictive Maintenance, & Industrial IoT

Primary Goal: Continuously evaluate bridge structural integrity using multi-sensor IoT networks to detect tilt, strain, crack expansion, and abnormal vibrations before critical failures occur.

1. Business Context & Problem Description

Bridges suffer from material fatigue, heavy traffic overloads, corrosion, and seismic activity. Periodic visual inspections are infrequent, labor-intensive, and unable to detect internal structural decay. Continuous IoT Structural Health Monitoring (SHM) enables real-time dynamic assessment, early flaw detection, and cost-effective predictive maintenance schedules.

2. Core Objectives

- Deploy synchronized wireless sensors across structural spans to measure strain, vibration, tilt, displacement, and environmental parameters.

- Aggregate high-frequency dynamic telemetry via edge gateways for real-time modal analysis.

- Detect structural anomalies by comparing active vibration frequencies and strain profiles against baseline physics models.

- Provide early alerts and long-term lifespan degradation trends on a central engineering platform.

3. Key Technical Challenges

- High-Frequency Data Synchronization: Precision time-stamping (sub-millisecond synchronization) across distributed sensor nodes across long bridge spans.

- Noise Separation: Distinguishing ambient traffic vibration from true structural stiffness degradation or cracking.

- Sensor Node Longevity: Ensuring uninterrupted node operation in remote outdoor locations using low-power design and energy harvesting.

4. Technical Requirements & Deliverables

- Structural Sensor Node Mesh: Wireless nodes incorporating tri-axial accelerometers, strain gauges, inclinometers, and temperature sensors.

- Edge Fast Fourier Transform (FFT) Module: Edge computing engine extracting modal frequencies, damping ratios, and peak accelerations.

- Cloud Predictive SHM Dashboard: Engineering platform featuring dynamic baseline comparison, anomaly notifications, and structural health scoring.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Sensor Timestamp Sync | Inter-node time synchronization drift <1 ms. |
| Anomaly Detection F1-Score | ≥0.95 in differentiating structural shifts from normal operational load variations. |
| Telemetry Packet Delivery | ≥99.9% reliable packet delivery over wireless mesh in high-interference environments. |

PS-09: Autonomous Floating Waste Collection & Water Monitoring Robot

Domain: Autonomous Marine Systems, Environmental Robotics, & Water Quality Sensing

Primary Goal: Automate surface plastic/debris harvesting in urban water bodies while simultaneously measuring water quality parameters and mapping spatial pollution hotspots.

1. Business Context & Problem Description

Floating plastic waste and organic debris choke urban lakes, rivers, and canals, leading to ecosystem degradation, blocked drainage, and severe public health risks. Manual cleaning boats are costly and impractical for continuous maintenance. An autonomous robotic vessel cleans surface debris continuously while generating comprehensive spatial water quality reports.

2. Core Objectives

- Autonomously navigate surface water bodies while avoiding static and dynamic obstacles.

- Capture and collect floating plastic, foam, and organic trash into an onboard containment basket.

- Sense key water quality indicators (pH, turbidity, temperature, Dissolved Oxygen, Total Dissolved Solids) in real time.

- Map GPS-tagged pollution heatmaps and stream operational analytics to a cloud monitoring portal.

3. Key Technical Challenges

- Unpredictable Aquatic Navigation: Navigating around floating debris fields, dense water weeds, shallow depths, and varying water currents.

- Sensory Bio-Fouling: Preventing algae and residue buildup from degrading probe accuracy during extended deployments.

- Power Management: Balancing motor propulsion and conveyor belt harvesting consumption against battery constraints.

4. Technical Requirements & Deliverables

- Autonomous Unmanned Surface Vessel (USV): Catamaran or twin-hull platform fitted with a conveyor/skimmer mechanism and GPS navigation.

- Environmental Sensor Payload: Water-quality probe unit measuring pH, turbidity, DO, TDS, and water temperature.

- Cloud Telemetry & Mapping Portal: Geospatial dashboard rendering vessel tracks, waste collection metrics, and water quality spatial maps.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Waste Harvesting Efficiency | ≥15 kg/hour of floating debris collected in target zones. |
| Waypoint Navigation Precision | Autonomous path tracking accuracy within ≤0.5 meters of target coordinates. |
| Sensor Profiling Correlation | ≥95% agreement with laboratory water sample testing standards. |

PS-10: Interactive AI Robotic Companion for Children

Domain: Consumer AI, Educational Robotics, & Human-Robot Interaction (HRI)

Primary Goal: Reduce passive screen time in children by driving engagement through an interactive, voice-first robotic companion providing physical games, storytelling, and personalized learning.

1. Business Context & Problem Description

Excessive screen time negatively impacts children's eyesight, physical health, sleep patterns, and social development. Parents struggle to find screen-free alternatives that hold a child's interest. An intelligent, voice-interactive robotic companion actively encourages physical movement, creative problem solving, and screen-free play habits.

2. Core Objectives

- Enable natural, age-appropriate conversational interactions using voice recognition, NLP, and speech synthesis.

- Deliver interactive learning experiences, including movement-based games, storytelling, dance, quizzes, and music.

- Personalize activity difficulty and content based on the child's age, interests, and developmental milestones.

- Track daily screen-free engagement and deliver actionable developmental insights to parents via a mobile app.

3. Key Technical Challenges

- Child Speech Recognition (Acoustics): Handling unique pitch, varying accents, incomplete grammar, and background noise typical of child speech.

- Low Response Latency: Maintaining natural, conversational cadence without long pauses during LLM/voice generation calls.

- Privacy & Safety Safeguards: Strict adherence to child data privacy standards (e.g., COPPA/GDPR-K) with on-device hardware privacy switches.

4. Technical Requirements & Deliverables

- Robotic Hardware Platform: Compact robotic shell with expressive display/LEDs, omnidirectional microphone array, speaker, and mobility actuators.

- Child-Centric Conversational Agent: Conversational AI pipeline with content filtering, speech synthesis, and adaptive gameplay logic.

- Parental Companion Mobile App: Mobile interface for setting screen-free goals, reviewing activity summaries, and managing content filters.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Child Speech Recognition WER | Word Error Rate (WER) ≤12% on child voice inputs. |
| Conversational Latency | End-to-end voice response turnaround <1.2 seconds. |
| Screen Time Reduction | Average ≥30% reduction in passive screen usage reported in family trials. |

PS-11: IoT Smart Cold Chain Temperature & Spoilage Monitor

Domain: Supply Chain, Food Safety, & Logistics

Primary Goal: Track ambient conditions in cold storage units during transport using low-cost sensor nodes to predict food spoilage and trigger immediate temperature alerts.

1. Business Context & Problem Description

Perishable items like dairy, pharmaceuticals, and fresh produce require continuous temperature and humidity control during transport. Breakdowns in refrigeration systems or unmonitored transit delays lead to massive financial losses and public health risks. Continuous, low-cost environmental logging prevents inventory loss before goods arrive damaged.

2. Core Objectives

- Continuously log temperature, humidity, and volatile organic compound (VOC) levels inside transport containers.

- Compute dynamic shelf-life decay using Arrhenius kinetic models on edge microcontrollers.

- Transmit real-time geolocation and environmental breach alerts over cellular or LoRa networks.

- Maintain an automated chain-of-custody compliance report for quality control audits.

3. Key Technical Challenges

- Power Optimization: Operating nodes on battery power for extended multi-day trips.

- Network Blackouts: Buffering telemetry data locally during cellular dead zones without data loss.

- Sensor Drift Calibration: Maintaining temperature accuracy within ±0.5∘C using basic sensors like the DHT22 or DS18B20.

4. Technical Requirements & Deliverables

- Hardware: ESP32 / Arduino Nano, DS18B20/DHT22 temperature sensors, MQ-135 air quality sensor, SIM800L GSM/GPS module.

- Firmware: Low-power sleep routines with local EEPROM/SD-card data buffering.

- Cloud Dashboard: Web portal rendering route maps, temperature graphs, and SMS alert configuration.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Battery Lifespan | ≥14 days of continuous monitoring on a single 18650 Li-ion cell. |
| Alert Latency | Breach notification sent within <30 seconds of network reconnection. |
| Data Integrity | 0% telemetry loss during offline buffering cycles. |

PS-12: Computer Vision Smart Parking Space Occupancy Engine

Domain: Smart Cities, Facility Management, & Mobility

Primary Goal: Detect available parking spots in open-air lots using a standard USB/IP webcam and edge vision models, eliminating the need for expensive per-spot ground sensors.

1. Business Context & Problem Description

Traditional smart parking systems rely on individual magnetic ground sensors installed under every single parking bay, incurring extreme hardware and installation costs. An overhead camera paired with computer vision can monitor dozens of parking bays simultaneously, offering an affordable, scalable alternative for municipal and commercial parking management.

2. Core Objectives

- Capture continuous video feeds of parking lots using standard CCTV or IP cameras.

- Map bounding boxes for individual parking slots onto the camera feed via a lightweight UI setup tool.

- Classify each slot as "Occupied" or "Vacant" using lightweight deep learning models.

- Stream real-time availability counts to a public web portal or digital signage driver.

3. Key Technical Challenges

- Occlusion & Perspective Glare: Vehicle shadows, rain reflections, and adjacent vehicle overhangs masking adjacent spots.

- Day-to-Night Transitions: Maintaining high classification accuracy under extreme lighting changes.

- Edge Processing Limits: Running inference across 30--50 parking slots per camera frame on a Raspberry Pi.

4. Technical Requirements & Deliverables

- Hardware: Raspberry Pi 4 / PC workstation, generic USB/RTSP IP Camera.

- Vision Pipeline: OpenCV region-of-interest (ROI) mapper paired with a lightweight image classifier (e.g., MobileNetV2 or custom CNN).

- Signage API Engine: Lightweight REST API streaming current slot capacity to mobile apps and LED displays.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Occupancy Precision | ≥96% accuracy across varying day and night lighting conditions. |
| Inference Rate | Frame processing interval ≤3 seconds for full slot refreshing. |
| Deployment Hardware Cost | Hardware setup cost <$100 per 30 parking spaces monitored. |

PS-13: Edge AI Acoustic Transformer Fault Inspector

Domain: Industrial Automation, Predictive Maintenance, & Manufacturing

Primary Goal: Detect early mechanical and electrical faults in motors, transformers, and pumps by analyzing operational acoustic signatures using a low-cost microphone and edge AI.

1. Business Context & Problem Description

Industrial equipment like transformers, induction motors, and compressors generate subtle changes in sound and hum frequencies prior to mechanical failure. Routine manual listener checks miss high-frequency bearing chatter or electrical arcing. Continuous acoustic monitoring using low-cost edge nodes flags abnormal operating frequencies before catastrophic breakdowns occur.

2. Core Objectives

- Capture acoustic emissions from heavy machinery using a standard MEMS microphone.

- Convert raw audio signals into Mel-spectrogram frequency representations in real time.

- Train an anomaly detection model (e.g., Autoencoder or MobileNet-Audio) to recognize normal operational hum vs. early wear patterns.

- Trigger maintenance warnings on local indicator LEDs and remote dashboards when sound signatures deviate from baseline.

3. Key Technical Challenges

- Background Factory Noise: Filtering out ambient plant noises, background voices, and nearby vehicle sounds.

- Acoustic Drift: Distinguishing benign load changes from structural component wear.

- On-Device Feature Extraction: Executing real-time Fast Fourier Transforms (FFT) and spectrogram generation on constrained microcontrollers.

4. Technical Requirements & Deliverables

- Hardware: ESP32 / Raspberry Pi Pico, INMP441 I2S MEMS Microphone module.

- Edge Audio Pipeline: On-device FFT and Mel-spectrogram generator running in C++/TinyML.

- Anomaly Engine: Unsupervised reconstruction loss model identifying acoustic deviations beyond baseline variance.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Fault Detection F1-Score | ≥0.92 in identifying mechanical wear patterns before failure. |
| False Positive Rate | <2% alerts triggered by ambient environmental plant noise. |
| Spectral Conversion Latency | Real-time audio window processing <200 ms. |

PS-14: Smart Agriculture Soil Moisture & Dynamic Irrigation System

Domain: Agritech, Resource Management, & Environmental Sensing

Primary Goal: Optimize agricultural water usage by automatically scheduling drip irrigation based on real-time soil moisture sensors, weather forecasts, and crop types.

1. Business Context & Problem Description

Over-watering damages crop yields, wastes groundwater reserves, and swells electricity costs for farmers. Conventional automated systems rely on basic timers that irrigate regardless of actual soil moisture or impending rainfall. A dynamic IoT system balances soil readings with local weather predictions to deliver precise water volumes.

2. Core Objectives

- Measure volumetric water content and temperature at root level using corrosion-resistant soil sensors.

- Fetch local open-source weather predictions to delay scheduled watering if rain is imminent.

- Actuate solenoid water valves via relay modules to irrigate specific farm zones autonomously.

- Monitor pump power usage and alert farmers to dry-run or pipe leak anomalies.

3. Key Technical Challenges

- Sensor Corrosion: Preventing fast oxidation in cheap capacitive/resistive soil probes.

- Power-Efficient Field Deployment: Operating outdoor field nodes using small solar panels and rechargeable batteries.

- Soil Heterogeneity: Calibrating sensor moisture thresholds across clay, sand, and loam soils.

4. Technical Requirements & Deliverables

- Hardware: ESP8266 / ESP32, Capacitive Soil Moisture Sensor v1.2, 5V Relay Module, 12V Solenoid Valve, 5W Solar Panel with TP4056 charger.

- Control Pipeline: Multi-variable decision engine integrating sensor moisture bounds and weather API triggers.

- Mobile Interface: WhatsApp/Telegram bot or simple web app for manual valve overrides and daily water logs.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Water Conservation | ≥30% reduction in overall water consumption vs. fixed timer systems. |
| Sensor Operational Life | Probe operation without signal degradation for ≥12 months. |
| System Uptime | 99% reliable remote control over low-bandwidth cellular/Wi-Fi links. |

PS-15: AI Computer Vision Worker PPE Compliance Monitor

Domain: Industrial Safety, Occupational Health, & Worksites

Primary Goal: Verify that workers on construction sites and manufacturing floors are wearing mandatory Personal Protective Equipment (PPE) like hardhats and safety vests using CCTV feeds.

1. Business Context & Problem Description

Workplace accidents on industrial sites frequently stem from non-compliance with Personal Protective Equipment (PPE) mandates. Manual visual monitoring by site managers is intermittent and difficult to maintain across large sites. An automated computer vision system continuously checks compliance and generates immediate site safety scores.

2. Core Objectives

- Stream video feeds from affordable RTSP security cameras across site access points.

- Detect workers and classify whether mandatory items (safety helmets, high-visibility vests) are correctly worn.

- Trigger local audio alarms when unhelmeted workers enter designated hazardous zones.

- Generate daily compliance reports highlighting non-compliance hotspots for safety managers.

3. Key Technical Challenges

- Distance & Resolution Limits: Detecting small hardhats on workers walking far from low-cost camera lenses.

- Extreme Occlusion: Workers carrying equipment or bending over masking safety vest coverage.

- Real-time Performance: Running object detection across multiple camera streams on low-cost hardware.

4. Technical Requirements & Deliverables

- Hardware: Raspberry Pi 4 / low-cost desktop GPU workstation, generic RTSP IP Cameras / USB Webcams.

- Vision Model: Fine-tuned YOLOv8-Nano / SSSD-Lite object detection model trained on hardhat/vest datasets.

- Safety Dashboard: Local dashboard logging violation snapshots, timestamped records, and floor-wise risk scores.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| PPE Detection Recall | ≥93% recall for hardhat and vest detection within 10 meters of camera. |
| Alert Generation Delay | Safety alarm trigger latency <1 second upon hazard zone entry. |
| Inference Speed | ≥15 FPS on edge compute hardware. |

PS-16: IoT Smart Energy Meter & Appliance Load Disaggregation

Domain: Smart Grid, Home Automation, & Energy Efficiency

Primary Goal: Monitor household current consumption in real time and estimate individual appliance usage (NIALM) using a non-invasive split-core current transformer sensor.

1. Business Context & Problem Description

High electricity bills often confuse consumers because standard energy meters show only aggregate monthly kilowatt-hours. Replacing every wall socket with a smart plug is expensive and intrusive. Non-Intrusive Load Monitoring (NILM) uses a single sensor clamped to the main electrical panel to identify individual appliance activity from total current waveforms.

2. Core Objectives

- Sample current and voltage waveforms at the main circuit breaker using a non-invasive current transformer.

- Calculate active power, reactive power, power factor, and cumulative energy usage on edge hardware.

- Identify appliance start/stop events (e.g., refrigerator cycles, AC compressor, microwave) using electrical signature processing.

- Push live power consumption metrics to a home dashboard or mobile app.

3. Key Technical Challenges

- Sampling Rate Constraints: High-speed analog-to-digital sampling needed to capture transient current spikes without overloading microcontrollers.

- Load Overlap: Disambiguating multiple resistive/inductive appliances operating simultaneously.

- Calibration Accuracy: Ensuring current readings match utility meter standards across varying voltage conditions.

4. Technical Requirements & Deliverables

- Hardware: ESP32, SCT-013-000 Non-invasive AC Current Transformer sensor, ZMPT101B Voltage Sensor module.

- DSP Pipeline: Real-time RMS current/voltage computation, active/reactive power calculation, and event detection logic.

- User Interface: Web dashboard displaying live power draws, daily cost metrics, and inferred appliance breakdowns.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Power Measurement Error | ≤3% error compared to calibrated commercial energy meters. |
| Appliance Event Detection | ≥85% F1-score for high-power appliance state identification (AC, Heater, Fridge). |
| Sampling Consistency | Continuous sensor sampling rate ≥1 kHz without buffer drops. |

PS-17: Smart LPG Gas Cylinder Leakage & Weight Monitoring System

Domain: Home Safety, Commercial Kitchens, & Asset Management

Primary Goal: Prevent catastrophic kitchen fires by detecting LPG gas leaks, automatically shutting off gas supply valves, and monitoring cylinder weight to predict depletion dates.

1. Business Context & Problem Description

Liquefied Petroleum Gas (LPG) leaks in home and restaurant kitchens present severe explosion and fire risks. Additionally, sudden gas depletion mid-cooking causes operational frustration. An affordable smart baseplate continuously tracks cylinder weight for timely refills while safeguarding against leaks by automatically cutting off gas flow.

2. Core Objectives

- Measure combustible gas concentrations (C4H10/C3H8) near kitchen floor levels in real time.

- Track LPG cylinder net weight using a strain-gauge load cell placed under the cylinder base.

- Actuate a motorized inline gas valve to shut off supply instantly upon detecting gas leaks.

- Send automated SMS/push notifications when gas level drops below 15% capacity.

3. Key Technical Challenges

- Sensor Drift & False Alarms: MQ gas sensors respond to humidity and alcohol fumes; filtering out false triggers is critical.

- Load Cell Creep: Preventing long-term mechanical drift in strain gauge readings under constant heavy weight (30--50 kg).

- Fail-Safe Operation: Ensuring the mechanical gas shutoff operates reliably even during power outages using battery backups.

4. Technical Requirements & Deliverables

- Hardware: ESP8266 / ESP32, MQ-6 Gas Sensor, HX711 Load Cell Amplifier with 50 kg Load Cell strain gauges, 12V Motorized Ball Valve, Piezo Buzzer.

- Firmware: Gas concentration threshold logic, moving-average weight filtering, and automatic valve triggering routines.

- Alert System: Local visual/audio alarm coupled with cloud notification services.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Leak Shutoff Latency | Valve closure initiated within <1.5 seconds of gas threshold breach. |
| Weight Prediction Accuracy | Remaining gas estimate accuracy within ±200 grams. |
| Fail-Safe Battery Life | System standby battery operational for ≥24 hours during mains outage. |

PS-18: Computer Vision Structural Concrete Crack Measurement Gauge

Domain: Infrastructure Inspection, Civil Engineering, & Quality Control

Primary Goal: Estimate surface crack width, orientation, and length on concrete walls and bridges using a smartphone or web camera paired with automated vision processing.

1. Business Context & Problem Description

Structural concrete inspection requires detecting hair-line surface cracks to assess building degradation. Traditional manual field measurement uses optical crack-width rulers, a process that is slow, subjective, and difficult to document digitally over time. An automated camera tool measures crack width with millimeter precision and logs spatial position.

2. Core Objectives

- Capture close-up images of concrete surfaces with an affordable camera or mobile phone lens.

- Segment crack boundaries from concrete texture noise using edge detection and image segmentation.

- Estimate physical crack width (in millimeters) using optical scaling algorithms and reference markers.

- Store inspection records with timestamped snapshots and severity tags in a digital catalog.

3. Key Technical Challenges

- Surface Texture Confusion: Distinguishing narrow concrete cracks from surface dirt, stains, or formwork joints.

- Sub-Millimeter Calibration: Achieving sub-millimeter measurement accuracy (0.1 mm width resolution) without expensive specialized lenses.

- Lighting Artifacts: Eliminating deep shadows inside cracks that distort edge detection calculations.

4. Technical Requirements & Deliverables

- Hardware: Raspberry Pi 4 with Camera Module / Android Smartphone / USB Digital Microscope.

- Vision Engine: OpenCV pipeline employing adaptive thresholding, Hessian matrix ridge filtering, and contour width profiling.

- Reporting Module: Mobile-friendly UI displaying color-coded crack masks, calculated maximum/average widths, and exportable PDF logs.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Width Estimation Error | Mean Absolute Error ≤0.15 mm compared to optical feeler gauge measurements. |
| Crack Segmentation Recall | ≥90% accurate identification of continuous crack lines. |
| Processing Time | Single image analysis and report generation <2 seconds. |

PS-19: IoT Smart Water Tank Level & Leakage Controller

Domain: Smart Buildings, Resource Conservation, & Facility Management

Primary Goal: Prevent urban water tank overflows, measure consumption trends, detect pipe leaks, and automate pump operations using non-contact sensors.

1. Business Context & Problem Description

Water tanks in residential and commercial buildings routinely overflow due to faulty float switches or delayed manual pump shutdowns, wasting thousands of liters of clean water. Unseen pipe leaks down the line cause structural damage and high water bills. An automated IoT controller monitors levels continuously and detects pipe anomalies.

2. Core Objectives

- Track overhead water tank levels in real time using non-contact ultrasonic or ToF sensors.

- Actuate water pump relays based on configurable upper/lower tank capacity bounds.

- Detect potential pipe leaks down the supply line by analyzing abnormal continuous flow patterns during non-peak hours.

- Display water availability metrics and daily pump runtime on a local display or mobile dashboard.

3. Key Technical Challenges

- Ultrasonic Echo Reflections: False distance readings caused by condensation drops on the sensor face or tank wall reflections.

- Pump Surge Protection: Preventing high-frequency relay toggling when surface waves ripple inside the tank.

- Wireless Range: Transmitting signals from roof-mounted tanks down through multiple concrete floor slabs to ground pumps.

4. Technical Requirements & Deliverables

- Hardware: ESP32 / Arduino, JSN-SR04T Waterproof Ultrasonic Distance Sensor, YF-S201 Water Flow Sensor, 30A Heavy-Duty Relay Module.

- Control Logic: Debounced moving-average level filter, dry-run pump safety shutdown, and night-time leak flow logic.

- Dashboard: Web/App interface displaying current percentage full, pump status, and historical daily usage graphs.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Overflow Prevention | 100% prevention of tank overflows during automated cycles. |
| Level Measurement Precision | Distance measurement accuracy within ±1 cm over a 3-meter depth range. |
| Leak Detection Sensitivity | Identifies continuous low-flow leaks down to ≥1.5 liters/minute. |

PS-20: AI Edge Computer Vision Smart Solar Panel Cleaner

Domain: Renewable Energy, Robotics, & Predictive Maintenance

Primary Goal: Detect dust and bird-dropping accumulation on solar photovoltaic (PV) panels using edge vision and trigger targeted automated wiper/brush mechanisms to restore efficiency.

1. Business Context & Problem Description

Dust, sand, and organic debris accumulation on solar panels can reduce photovoltaic power output by up to 20--30%. Cleaning entire solar arrays on a fixed time schedule wastes water and accelerates mechanical wear on cleaning rigs. An AI camera system assesses actual surface soiling and triggers cleaning cycles only when necessary.

2. Core Objectives

- Inspect solar panel surfaces using an automated overhead camera or mobile crawler unit.

- Classify surface soiling coverage into clear, light dust, heavy dirt, or localized hotspots (e.g., bird droppings).

- Trigger automated motorized wiper/water spray mechanisms across dirty panels.

- Log estimated power restoration metrics post-cleaning cycle.

3. Key Technical Challenges

- Direct Sun Glare: Processing high-contrast reflection images taken in direct sunlight.

- Hotspot Detection: Identifying small, concentrated contaminants (like droppings) that cause localized panel overheating.

- Low-Power Edge Hardware: Operating inspection and motor actuation on outdoor solar-powered edge hardware.

4. Technical Requirements & Deliverables

- Hardware: ESP32-CAM / Raspberry Pi Zero 2W, 12V DC Motor Driver (L298N), Linear Actuator/Wiper motor assembly, 5V Relay.

- Vision Algorithm: Lightweight image segmentation model (or OpenCV texture entropy analysis) measuring dirty vs. clean surface ratio.

- Telemetry Dashboard: Remote cloud platform logging cleaning frequencies, estimated efficiency gains, and motor status.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Soiling Segmentation Accuracy | ≥90% correlation with manual cleanliness ratings. |
| Power Output Recovery | Restores PV efficiency to ≥95% of clean rating post-trigger. |
| Inference Compute | Image analysis completes within <1 second on ESP32-CAM/Raspberry Pi. |

PS-21: Smart Indoor Air Quality & Automated HVAC Filter Monitor

Domain: Building Management, Environmental Health, & HVAC Automation

Primary Goal: Monitor particulate matter (PM2.5,PM10), carbon dioxide (CO2), and humidity in indoor working spaces, dynamically controlling fresh air intake and alerting to clogged filters.

1. Business Context & Problem Description

Poor indoor air quality (IAQ) causes fatigue, lowered cognitive function, and respiratory issues among indoor workers. Running HVAC fans continuously wastes electricity, while delayed filter replacements cause system strain and poor filtration. Smart IoT nodes sense IAQ trends, regulate ventilation fans dynamically, and issue automated maintenance alerts when filters clog.

2. Core Objectives

- Sample indoor particulate matter (PM2.5,PM10), CO2 levels, temperature, and relative humidity.

- Sense differential pressure across HVAC air filters to detect clogging.

- Control ventilation fans dynamically using PWM signals to adjust air exchange based on occupant load and gas build-up.

- Push live air quality index (AQI) values to digital wall displays and building facility dashboards.

3. Key Technical Challenges

- CO2 Sensor Calibration: Non-Dispersive Infrared (NDIR) sensors require baseline calibration routines to prevent drift.

- Low-Noise Fan Control: Smooth PWM speed transitions without generating acoustic motor hum.

- Dynamic Thresholding: Balancing energy conservation against target indoor AQI limits.

4. Technical Requirements & Deliverables

- Hardware: ESP32, PMS5003 Dust Particulate Sensor, MH-Z19B NDIR CO2 Sensor, BME280 Temp/Humidity Sensor, MOSFET PWM fan driver.

- Control Loop: PID loop modulating fan speed relative to real-time CO2 PPM and PM concentrations.

- Facility UI: Real-time web portal displaying floor-wise AQI heatmaps and filter lifecycle health bars.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| CO2 Regulation Speed | Reduces indoor CO2 below 1000 PPM within <10 minutes of occupancy spike. |
| HVAC Energy Savings | ≥20% energy reduction compared to static full-speed fan operation. |
| Filter Clog Detection | Identifies filter obstruction within ±5% differential pressure threshold. |

PS-22: IoT RFID & Smart Scale Inventory Automated Bin System

Domain: Manufacturing, Retail Stockrooms, & Warehouse Management

Primary Goal: Track bin-level inventory of small hardware parts (screws, electronic components, fittings) using weight scale sensors and RFID tags to trigger auto-replenishment.

1. Business Context & Problem Description

Stockouts of small low-cost items (e.g., bolts, resistors, fasteners) on assembly lines halt entire manufacturing shifts. Manual stock checks are error-prone and time-consuming. Equipping storage bins with integrated strain-gauge scales and RFID readers provides real-time, automated inventory counts without manual logging.

2. Core Objectives

- Measure bin weight continuously to compute precise unit counts of uniform hardware components.

- Read RFID tags attached to inventory bins to automatically associate item metadata with specific shelf slots.

- Trigger purchase requisitions automatically when component counts drop below predefined reorder thresholds.

- Display stock counts on OLED screen modules mounted on each shelf bin.

3. Key Technical Challenges

- Single-Unit Weight Resolution: Accurately detecting small weight changes when single components weigh under 2--3 grams.

- Load Cell Calibration: Auto-zeroing tare weight variations caused by changing bin containers or dust accumulation.

- Bus Multiplexing: Interfacing dozens of weight scale modules to a single controller node across storage racks.

4. Technical Requirements & Deliverables

- Hardware: ESP32 / Raspberry Pi, HX711 Load Cell Amplifiers with 1--5 kg Load Cells, RC522 RFID Reader, 0.96" I2C OLED Display.

- Inventory Engine: Piece-weight calibration routine, low-stock threshold triggers, and inventory change event streaming.

- Warehouse Portal: Web application managing inventory balances, SKU mappings, and purchase order triggers.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Unit Count Precision | Accuracy within ±1 item for individual component weights down to 1.5 grams. |
| Stockout Warning SLA | Reorder alert generated within <5 seconds of stock dipping below threshold. |
| Hardware Node Cost | Hardware cost <$15 per smart bin slot monitored. |

PS-23: Computer Vision Smart Pest & Disease Leaf Classifier

Domain: Precision Farming, Crop Protection, & Mobile AI

Primary Goal: Detect and classify crop leaf diseases and pest infestations in real time using a low-cost camera module and edge AI, providing immediate treatment guidance to farmers.

1. Business Context & Problem Description

Plant diseases and pest attacks cause major agricultural yield losses. Farmers often identify infection stages too late or misapply inappropriate chemical pesticides. A handheld or tractor-mounted camera tool equipped with an edge vision model identifies specific plant pathologies early and suggests precise organic or chemical remedies.

2. Core Objectives

- Capture high-resolution images of crop leaves using an onboard camera or smartphone camera.

- Segment diseased leaf areas and classify pathologies (e.g., powdery mildew, leaf rust, blight, pest damage).

- Display localized disease names and recommended treatment steps on a simple screen interface.

- Geotag infection locations on a map to track disease spread across farm fields.

3. Key Technical Challenges

- Complex Outdoor Backgrounds: Isolating target leaves from background soil, weeds, and overlapping foliage.

- Visual Pathology Similarities: Disambiguating early-stage fungal infections with similar visual spots.

- Edge Deep Learning Memory: Executing multi-class image classification on microcontrollers with constrained RAM.

4. Technical Requirements & Deliverables

- Hardware: ESP32-CAM / Raspberry Pi 4 with Camera Module, TFT Display Shield.

- Vision Model: Fine-tuned MobileNetV3 / EfficientNet-Lite model trained on agricultural leaf disease datasets (e.g., PlantVillage).

- On-Device Application: Lightweight UI rendering visual bounding boxes, infection severity ratings, and treatment cards.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Disease Classification F1 | ≥91% top-1 accuracy across 10 common crop leaf diseases. |
| Image Processing Latency | <1.5 seconds on edge compute hardware (Raspberry Pi/ESP32-S3). |
| Model Size | Quantized model binary size <15 MB for local device storage. |

PS-24: Smart Electrical Distribution Box Thermal & Overload Monitor

Domain: Electrical Safety, Building Automation, & Fire Prevention

Primary Goal: Prevent electrical panel fires by continuously monitoring breaker temperatures using non-contact infrared thermal sensors and current transducers.

1. Business Context & Problem Description

Electrical distribution boards in commercial and industrial buildings are major sources of structural fires due to loose wire connections, phase imbalances, and circuit overloads. Thermal imaging inspections are typically performed manually once a year. Continuous IoT thermal tracking catches loose connections and thermal runaway long before smoke develops.

2. Core Objectives

- Measure surface temperature across electrical circuit breaker terminals using array-based thermal infrared sensors.

- Measure current flow across phases using non-contact current sensors to flag phase imbalance.

- Sound local buzzer alarms and trip main safety contactors if terminal temperatures exceed critical thresholds (>85∘C).

- Push thermal heatmaps and load metrics to a facility safety portal.

3. Key Technical Challenges

- Low-Resolution Thermal Segmentation: Extracting accurate component temperatures from low-cost array sensors (8×8 or 32×24 pixels).

- Enclosure Installation: Mounting sensor arrays securely inside crowded breaker boxes without interfering with high-voltage wiring.

- Noise Filtering: Distinguishing normal load heating from dangerous thermal runaway caused by high contact resistance.

4. Technical Requirements & Deliverables

- Hardware: ESP32, AMG8833 (8×8) or MLX90640 (32×24) IR Thermal Array Sensor, SCT-013 Current Sensors, Relay Contactor Trigger, Piezo Buzzer.

- Processing Engine: Bilinear interpolation of thermal arrays, dynamic hotspot tracking, and temperature gradient analysis.

- Alert System: Visual thermal heatmap UI, local alarm output, and instant SMS/Email notification integration.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Hotspot Detection Accuracy | Identifies local temperature spikes within ±2.0∘C of reference thermal cameras. |
| Alarm Trigger Latency | Emergency cutoff relay triggered within <500 ms of reaching critical heat limits. |
| Sensor Cost Savings | Hardware component cost <$40 per electrical box monitored. |

PS-25: IoT Smart Waste Bin Fill-Level & Route Optimization Engine

Domain: Smart Cities, Municipal Operations, & Logistics

Primary Goal: Optimize garbage truck collection routes by continuously monitoring public trash bin fill levels and tilt status using low-cost ultrasonic sensor nodes.

1. Business Context & Problem Description

Municipal waste collection trucks follow fixed daily routes, frequently visiting empty public trash bins while overflowing bins in high-traffic areas go unserviced. This leads to wasted fuel, higher vehicular emissions, and unhygienic street conditions. Real-time bin level tracking optimizes collection routes so trucks visit only bins requiring immediate emptying.

2. Core Objectives

- Measure trash level height inside municipal bins using battery-powered ultrasonic sensors.

- Detect bin tipping or vandalism events using internal tilt/accelerometer sensors.

- Transmit bin status updates to a central platform once or twice daily over low-power cellular/LoRa links.

- Generate dynamic, prioritized collection routes for waste vehicle drivers based on real-time bin priority queues.

3. Key Technical Challenges

- Non-Uniform Trash Surfaces: Handling uneven garbage disposal profiles (e.g., crumpled paper, bottles) that scatter ultrasonic signals.

- Ultra-Low Power Operations: Achieving multi-year battery life on field nodes exposed to harsh outdoor weather.

- Rugged Enclosures: Protecting sensors from humidity, trash residue, foul odors, and physical vandalism.

4. Technical Requirements & Deliverables

- Hardware: ESP32-CAM / ATmega328P / Low-Power MCU, HC-SR04 / JSN-SR04T Waterproof Ultrasonic Sensor, MPU6050 Accelerometer, SIM7000E NB-IoT/LTE-M module.

- Firmware: Deep-sleep firmware waking nodes on timed intervals or tilt interrupts.

- Route Engine: Cloud portal rendering fill-level maps and computing shortest-path vehicle routes (TSP algorithm).

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Battery Operational Life | ≥2 years operational lifespan on 2x AA Lithium batteries (2 transmissions/day). |
| Distance Error Rate | Depth measurement accuracy within ±3 cm inside standard 240L bins. |
| Collection Logistics Savings | ≥25% reduction in truck route mileage compared to fixed schedules. |

PS-26: Computer Vision Smart Belt Conveyor Sorting System

Domain: Logistics, E-Commerce Packaging, & Factory Automation

Primary Goal: Identify, classify, and sort packages moving on a conveyor belt based on shape, color, or barcode using a webcam and servo-driven diverter arms.

1. Business Context & Problem Description

High-throughput distribution hubs spend significant resources manually sorting packages, parcels, and manufactured parts into destination chutes. Industrial sorting systems are cost-prohibitive for small and medium enterprises. An affordable, camera-driven conveyor sorter automates item routing using low-cost microcontrollers and servomotors.

2. Core Objectives

- Detect objects arriving on a moving conveyor belt using a camera feed.

- Classify objects by dimensional footprint, color signature, or visual barcode/QR code data.

- Calculate object travel speed and timing to actuate mechanical servo diverters at the exact arrival moment.

- Track hourly sorting counts and error rates on a local control panel.

3. Key Technical Challenges

- Motion Blur: Processing sharp object frames from moving conveyor belts under standard indoor lighting.

- Precise Actuation Timing: Synchronizing visual detection events with mechanical servo arm triggers.

- Variable Object Geometry: Classifying soft-packaged mailers, polybags, and irregular cardboard boxes accurately.

4. Technical Requirements & Deliverables

- Hardware: Raspberry Pi 4 / PC workstation, High-frame-rate USB Webcam, PCA9685 Servo Driver board, TowerPro MG996R Servomotors / Solenoid actuators, Conveyor assembly.

- Vision Pipeline: OpenCV contour extraction, color histogram matching, 1D/2D Barcode decoder module, and tracking algorithm.

- Control Software: High-precision timer thread issuing servo actuation commands mapped to conveyor speed.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Sorting Accuracy | ≥97% correct package diversion at belt speeds up to 0.5 meters/second. |
| Item Processing Rate | Handles ≥40 items per minute continuously. |
| System Hardware Cost | Total sorting cell hardware cost <$200 (excluding frame/belt). |

PS-27: IoT Smart Vibration & Health Monitor for Rotating Machinery

Domain: Plant Maintenance, Heavy Industry, & Asset Performance

Primary Goal: Track mechanical vibration spectra on pump shafts, fans, and gearboxes using MEMS accelerometers to detect bearing wear, shaft imbalance, and misalignment.

1. Business Context & Problem Description

Unplanned downtime caused by bearing failure or shaft misalignment in industrial machinery leads to costly production halts. Traditional manual vibration audits using handheld meters are performed intermittently, missing rapid wear escalation. Continuous vibration tracking using surface-mounted MEMS sensors catches mechanical wear in its earliest stages.

2. Core Objectives

- Measure high-frequency tri-axial acceleration vibrations on motor casings and bearing housings.

- Compute peak acceleration, velocity RMS, and Fast Fourier Transform (FFT) frequency spectra on edge hardware.

- Compare measured RMS values against ISO 10816 industrial vibration severity standards.

- Send automated alert flags when vibration levels enter "Unsatisfactory" or "Unacceptable" operating zones.

3. Key Technical Challenges

- Sampling Bandwidth Limits: Sampling MEMS accelerometers at high frequencies (≥5 kHz) to capture subtle bearing pass frequencies.

- Sensor Mounting Rigidity: Ensuring firm mechanical coupling (magnetic or stud mount) to eliminate vibration dampening.

- Data Volume Reduction: Processing raw vibration streams on edge nodes to stream only spectral summaries and health indices.

4. Technical Requirements & Deliverables

- Hardware: ESP32, ADXL345 / MPU6050 High-Speed Tri-Axial Accelerometer (I2C/SPI), Magnetic Mounting Base, OLED status display.

- Edge Processing Engine: On-device Fast Fourier Transform (FFT) library extracting peak vibration frequencies and overall RMS velocity.

- Maintenance Portal: Dashboard displaying real-time vibration wave plots, FFT spectral peaks, and ISO status gauges.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| FFT Frequency Precision | Vibration peak frequency detection within ±2 Hz accuracy. |
| ISO Standard Alignment | 100% agreement with ISO 10816 vibration classification thresholds. |
| Data Reduction Ratio | Edge FFT processing reduces raw streaming telemetry bandwidth by ≥95%. |

PS-28: Edge Computer Vision Smart Traffic Light Controller

Domain: Intelligent Transportation Systems, Urban Planning, & Traffic Management

Primary Goal: Dynamically adjust traffic light green-time durations at intersections based on real-time vehicle queue lengths measured by edge AI camera nodes.

1. Business Context & Problem Description

Static timer-based traffic lights cause unnecessary traffic congestion, fuel waste, and emergency vehicle delays because signal timings do not adjust to real-time traffic demand. Equipping intersections with camera-based edge vision nodes measures vehicle counts per lane dynamically, adjusting signal timing to clear congested lanes efficiently.

2. Core Objectives

- Monitor vehicle queues across intersection lanes using overhead camera feeds.

- Count vehicles per lane and classify vehicle types (cars, buses, motorcycles, trucks) using edge object detection.

- Compute optimal green-light duration dynamically based on real-time lane congestion ratios.

- Detect incoming emergency vehicles (ambulances, fire engines) to grant immediate green wave priority.

3. Key Technical Challenges

- Occlusion in Dense Queues: Accurately counting bumper-to-bumper vehicles in packed lane queues.

- Low-Latency Switching: Executing object detection and light control logic fast enough to adjust cycles smoothly.

- Fail-Safe Operation: Defaulting instantly to standard fixed timer hardware logic if camera or compute hardware fails.

4. Technical Requirements & Deliverables

- Hardware: Raspberry Pi 4 / NVIDIA Jetson Nano, RTSP IP Cameras / USB Webcams, 4-Channel Relay Board driving traffic signal lamps.

- Vision Pipeline: Lightweight object detection model (YOLOv8-Nano / MobileNet-SSD) trained on vehicle datasets.

- Signal Scheduling Engine: Dynamic green-time allocation algorithm balancing lane priority queues and maximum wait times.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Vehicle Counting Precision | ≥92% accuracy in dense traffic queues across all lanes. |
| Average Wait Time Reduction | ≥20% reduction in intersection delay times compared to fixed timers. |
| Fail-Safe Recovery Latency | Fallback to hardware timer within <1 second of vision software error. |

PS-29: Smart Water Quality Sensing Station for Inland Aquafarms

Domain: Aquaculture, Smart Farming, & Environmental Protection

Primary Goal: Monitor pH, Dissolved Oxygen (DO), temperature, and water clarity in fish and shrimp ponds continuously to automate aerator pumps and prevent mass die-offs.

1. Business Context & Problem Description

Commercial fish and shrimp farming requires maintaining strict water quality limits. Sudden drops in Dissolved Oxygen (DO) levels or extreme pH fluctuations cause high aquatic mortality within hours. Manual chemical test kits are used irregularly, often failing to detect dangerous night-time oxygen drops. Continuous IoT sensing protects stock and automates aeration routines.

2. Core Objectives

- Submerge sensor probes to measure water pH, Dissolved Oxygen (DO), temperature, and Total Dissolved Solids (TDS) continuously.

- Actuate pond paddle-wheel aerator motors automatically via heavy-duty relays when oxygen levels drop.

- Alert fish farmers via automated phone calls/SMS when parameters breach critical aquatic safety thresholds.

- Stream historical water quality data to a central cloud dashboard for feeding and harvest optimization.

3. Key Technical Challenges

- Probe Bio-Fouling: Preventing algae growth on sensitive pH and DO glass electrodes during extended submersion.

- Signal Isolation: Eliminating electrical ground loops in water that distort analog sensor readings from multiple probes.

- Sensor Calibration Routine: Simplifying multi-point calibration procedures for non-technical farm operators.

4. Technical Requirements & Deliverables

- Hardware: ESP32, Analog pH Sensor kit, Analog Dissolved Oxygen Sensor kit, DS18B20 Waterproof Temp Sensor, Analog TDS Sensor, Heavy-Duty Relay Contactor.

- Analog Conditioning: Signal isolation boards preventing inter-probe electrical interference.

- Monitoring Portal: Web platform and SMS integration displaying live water health metrics, aeration logs, and calibration wizards.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Automated Aerator Trigger | Aerator motor activated within <10 seconds of DO dropping below 4.0 mg/L. |
| Sensor Measurement Precision | Temperature accuracy ±0.5∘C; pH accuracy ±0.2 pH. |
| Bio-Fouling Operational Limit | Probes operate reliably without manual cleaning for ≥14 days. |

PS-30: AI Speech-Guided Edge Voice Assistant for Machine Operation

Domain: Industrial HMI, Accessibility, & Hands-Free Automation

Primary Goal: Control industrial machines, lathes, and assembly equipment using local, offline voice commands processed completely on an edge microcontroller without cloud latency or privacy risks.

1. Business Context & Problem Description

Operators running industrial machinery often have both hands occupied with parts or safety tools, making manual button/touchscreen interaction slow and awkward. Cloud-based voice assistants (e.g., Alexa, Google Assistant) are unacceptable in industrial environments due to network latency, cloud dependency, and privacy risks. A fully offline edge voice controller enables secure, hands-free operation.

2. Core Objectives

- Capture operator voice commands using a noise-canceling directional microphone array.

- Process speech recognition completely offline on an edge microcontroller/processor node.

- Recognize specific operational wake words and command phrases (e.g., "Start Conveyor", "Emergency Pause", "Set Speed 5").

- Trigger digital output pins and motor relays to actuate machinery directly with sub-second response times.

3. Key Technical Challenges

- Heavy Factory Ambient Noise: Filtering out background motor hum, mechanical clatter, and adjacent voices (>75 dB environments).

- Zero Cloud Latency: Executing Speech-to-Text (STT) and intent matching locally within tight microchip memory constraints.

- Safety & False Command Prevention: Preventing unintended machine actuation caused by misheard background speech.

4. Technical Requirements & Deliverables

- Hardware: ESP32-S3 / Raspberry Pi 4, I2S MEMS Microphone Array module (e.g., INMP441 / ReSpeaker 2-Mics Shield), 4-Channel Relay Output module.

- Offline Speech Engine: Lightweight keyword spotting (KWS) and intent parsing engine (e.g., MicroSpeech / PocketSphinx / Picovoice Porcupine local runtime).

- Machine Control Interface: Direct GPIO hardware output map triggering relays and motor drivers accompanied by audio confirmation beeps.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Command Keyword Accuracy | ≥94% correct command recognition in noisy industrial environments (75 dB). |
| Offline Latency | Command execution trigger delay <400 ms post-phrase utterance. |
| False Trigger Rate | Zero false command triggers during 8 hours of ambient background factory playback. |

PS-31: Non-Invasive Water Pipeline Corrosion & Flow Analytics

Domain: Smart Infrastructure, Asset Management, & Utilities

Primary Goal: Estimate internal pipe degradation, scale buildup, and flow velocity non-invasively using surface-mounted acoustic/vibration sensors and edge signal processing.

1. Business Context & Problem Description

Municipal and industrial water networks suffer from unseen internal pipe corrosion and mineral scaling, leading to catastrophic bursts or restricted flow rates. Excavating or cutting open pressurized pipes to inspect their interior is prohibitively expensive and disruptive. Strapping low-cost piezo acoustic sensors onto external pipe walls allows data scientists to infer internal structural degradation and fluid flow dynamics from structural resonance changes.

2. Core Objectives

- Capture acoustic resonance and flow-induced vibration signals from external pipe walls non-invasively.

- Isolate structural resonance frequencies to detect wall thinning, internal pitting, and heavy mineral scale accumulation.

- Estimate fluid flow rates continuously from flow-generated turbulences.

- Stream pipe health indices and maintenance alerts to a centralized utility management dashboard.

3. Key Technical Challenges

- Acoustic Attenuation: Disambiguating fluid turbulence noise from structural resonance shifts caused by wall thinning.

- Pipe Material Variability: Calibrating baseline acoustic propagation velocities across PVC, galvanized iron, and copper pipes.

- Low-Power Continuous Sensing: Executing Fast Fourier Transforms (FFT) on low-power microcontrollers without draining field battery packs.

4. Technical Requirements & Deliverables

- Hardware: ESP32, Piezoelectric Contact Vibration Sensors (e.g., LDT0-028K), MAX9814 Preamplifier Modules, Strap-on Pipe Clamps.

- Signal Pipeline: Bandpass filtering, Fast Fourier Transform (FFT) spectrum extraction, and resonance shift detection logic.

- Utility Portal: Dashboard displaying pipe wall health scores, estimated flow rates, and maintenance dispatch triggers.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Corrosion/Scale Estimation Accuracy | ≥85% correlation with ultrasonic thickness gauge measurements. |
| Non-Invasive Installation Time | <5 minutes per sensing node without pipe interruption. |
| Edge FFT Computation Time | Spectral analysis execution time <150 ms on ESP32. |

PS-32: Edge Computer Vision Livestock Health & Rumination Tracker

Domain: Smart Farming, Livestock Management, & AgTech

Primary Goal: Track livestock rumination cycles, jaw movement, and early lethargy signs in dairy cattle using an overhead low-cost webcam and edge computer vision.

1. Business Context & Problem Description

Rumination (cud-chewing) duration is the single most critical early indicator of metabolic health, stress, and impending illness in dairy cows. Traditional monitoring relies on expensive wearable smart collars or manual observation by farm hands. An overhead edge vision camera installed above feeding stalls automatically tracks head/jaw motions across multiple cattle simultaneously, democratizing precision livestock monitoring.

2. Core Objectives

- Track cattle head locations and jaw movement frequencies in feeding stalls using overhead camera feeds.

- Classify real-time activities into eating, ruminating, resting, or abnormal lethargic behavior.

- Calculate daily cumulative rumination time per tagged cow stall.

- Send automated mobile alerts to farmers when a cow's rumination time drops by ≥20% below its baseline.

3. Key Technical Challenges

- Subtle Jaw Motion Detection: Detecting fine repetitive jaw movements from a distance under varying stable illumination.

- Animal Movement & Occlusion: Maintaining cow tracking when animals shift positions or overlap in stall views.

- Edge Resource Limits: Running multi-object head tracking and temporal motion analysis on low-cost hardware.

4. Technical Requirements & Deliverables

- Hardware: Raspberry Pi 4 / PC Workstation, Generic RTSP IP Camera / USB Webcam.

- Vision Model: Optical flow estimation or lightweight pose/keypoint detector (e.g., MediaPipe/YOLO-Pose) tracking head/mouth movement frequencies.

- Alert Portal: Mobile-friendly dashboard rendering cow health logs, daily rumination charts, and anomaly flags.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Rumination Detection Precision | ≥90% correlation with manual observational rumination counts. |
| Illumination Robustness | Sustained tracking accuracy in low-light indoor barn conditions (≥20 lux). |
| Early Disease Warning SLA | Flags health anomalies ≥24 hours before clinical symptoms appear. |

PS-33: Smart Acoustic Transformer Arcing & Corona Discharge Monitor

Domain: Power Utilities, Electrical Grid Safety, & Predictive Maintenance

Primary Goal: Detect silent electrical arcing and corona discharge in high-voltage substations and electrical cabinets using ultrasonic acoustic sensors and edge pattern recognition.

1. Business Context & Problem Description

High-voltage electrical transformers, switchgears, and distribution panels experience insulation breakdown that manifests as high-frequency electrical arcing or corona discharge before catastrophic explosion or blackout events occur. Because corona discharge emits sound primarily in the ultrasonic spectrum (20--100 kHz), it is completely silent to human ears. Low-cost ultrasonic microphones capture these hidden emissions early.

2. Core Objectives

- Continuous monitoring of electrical enclosures using high-frequency ultrasonic acoustic transducers.

- Down-mix and analyze ultrasonic emissions (20--80 kHz) on edge hardware using heterodyne or digital signal processing algorithms.

- Distinguish dangerous electrical arcing/corona signatures from benign mechanical noise (cooling fans, wind).

- Trigger immediate circuit breaker isolation relays and alert utility operators upon arcing detection.

3. Key Technical Challenges

- High-Frequency Sampling Requirements: Interfacing ultrasonic transducers requiring high-speed analog sampling without high-cost Oscilloscopes or DAQs.

- Acoustic Noise Rejection: Eliminating ambient high-frequency acoustic noise generated by nearby machinery or pressure leaks.

- Arcing vs. Corona Differentiation: Classifying the severity of insulation breakdowns based on acoustic pulse patterns.

4. Technical Requirements & Deliverables

- Hardware: ESP32-S3 / Raspberry Pi Pico, Ultrasonic MEMS Microphone / Transducer (e.g., Knowles SPU0410LR5H), 12V Relay Contactor.

- DSP Framework: On-device Fast Fourier Transform (FFT) spectral power density analyzer targeting 20--60 kHz bands.

- Grid Safety Portal: Central web platform logging discharge intensity decibels, timestamped event logs, and breaker status.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Arcing Detection Accuracy | ≥96% true positive detection of partial discharge / arcing events. |
| False Alarm Rate | <1% false triggers caused by ambient plant or environmental noise. |
| Response Latency | Emergency shutdown signal generated within <100 ms of arc initiation. |

PS-34: Optical Computer Vision Smart Counterfeit Currency & Document Inspector

Domain: Banking, Retail Operations, & Loss Prevention

Primary Goal: Verify paper currency authenticity, security thread patterns, and document UV watermark signatures using an off-the-shelf camera module and multi-spectral LED lighting.

1. Business Context & Problem Description

Counterfeit currency notes and forged identity documents cause significant financial losses for small retailers, transport hubs, and banking kiosks. Professional UV counterfeit detectors are costly and require trained human operators to manually verify watermarks under specialized light. An automated, low-cost multi-spectral camera module automatically validates notes and credentials within seconds.

2. Core Objectives

- Illuminate banknotes or documents sequentially under White, Ultraviolet (UV-A), and Infrared (IR) LEDs using micro-controllers.

- Capture multi-spectral image frames using a standard camera module without IR-cut filters.

- Analyze fluorescent security threads, micro-printing, and optical variable ink patterns using computer vision.

- Display instant "PASS / FAIL / COUNTERFEIT" flags on an onboard display screen.

3. Key Technical Challenges

- Multi-Spectral LED Synchronization: Precisely synchronizing multi-color LED switching with camera frame exposure triggers.

- Image Alignment & Registration: Compensating for slight manual placement offsets of banknotes under the camera.

- Wear and Tear Degradation: Distinguishing truly counterfeit notes from genuine, worn, dirty, or folded currency notes.

4. Technical Requirements & Deliverables

- Hardware: ESP32-CAM / Raspberry Pi Zero 2W (NoIR Camera), UV-A (365 nm) LEDs, IR (850 nm) LEDs, White LEDs, 0.96" OLED Display.

- Vision Pipeline: Multi-frame feature extraction checking UV fluorescence intensity, watermark contours, and dimensional aspects.

- Retail UI Engine: Standalone device firmware running complete local inference with zero internet requirement.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Counterfeit Detection Accuracy | ≥98% correct identification of fake notes and altered document features. |
| Verification Speed | Complete multi-spectral scan and decision output within <1.5 seconds. |
| Hardware Enclosure Cost | Total component hardware cost <$25. |

PS-35: IoT Smart Grain Storage Silo Temperature & Humidity Probe Vector

Domain: Agriculture Storage, Food Security, & Supply Chain

Primary Goal: Prevent grain spoilage and fungal rot inside agricultural storage silos using a cable-suspended array of digital temperature and humidity sensors.

1. Business Context & Problem Description

Post-harvest grain loss in storage silos due to moisture accumulation, fungal rot, and insect infestation affects millions of tons of food annually. Moisture pockets create localized "hotspots" where grain fermenting produces destructive heat. Suspending a low-cost multiplexed cable array of digital temperature/humidity sensors down silo centers enables early detection of internal spoilage pockets.

2. Core Objectives

- Measure multi-depth temperature and relative humidity continuously along vertical grain column profiles.

- Compute equilibrium grain moisture content (EMC) using psychrometric equations on edge microcontrollers.

- Identify localized thermal hotspots indicating early fungal fermentation or pest activity.

- Actuate aeration fans automatically to equalize silo moisture and dissipate internal heat build-up.

3. Key Technical Challenges

- Dust & Mechanical Wear: Protecting low-cost sensor nodes from high abrasion, grain dust, and mechanical pressure during loading.

- One-Wire Bus Scalability: Reading dozens of digital sensors connected across long vertical cables (5--15 meters) without data corruption.

- Early Hotspot Localization: Detecting localized thermal spikes before heat diffuses across large surrounding grain volumes.

4. Technical Requirements & Deliverables

- Hardware: ESP32, Waterproof DS18B20 Temperature Sensors / SHT30 Sensors multiplexed along suspended cables, 12V Relay driving aeration fans.

- Firmware: One-Wire bus protocol reader, 3D temperature matrix interpolation, and automatic fan control routines.

- Silo Dashboard: Web interface rendering vertical temperature/humidity heatmaps per silo with automated alert logs.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Hotspot Detection Sensitivity | Identifies localized temperature increases of ≥2.0∘C above baseline. |
| Post-Harvest Loss Reduction | Prevents grain spoilage events by ≥90% in monitored silos. |
| Data Line Bus Reliability | 0% sensor packet loss over 15-meter sensor cable drops. |

PS-36: Smart Optical Sensor Fluid Contamination & Turbidity Analyzer

Domain: Industrial Manufacturing, Automotive Maintenance, & Environmental

Primary Goal: Detect particulate degradation, water contamination, and clarity loss in industrial lubricants, hydraulic oils, and coolant fluids using a low-cost optical sensor module.

1. Business Context & Problem Description

Hydraulic fluid, engine oil, and industrial coolants break down over time due to metallic wear debris, soot, chemical oxidation, and water ingress. Running machinery with degraded fluids leads to severe mechanical wear and catastrophic pump/engine failures. An inline optical sensor measuring multi-wavelength light transmission provides real-time fluid health tracking.

2. Core Objectives

- Measure light attenuation and scattering across fluid channels using multi-wavelength LEDs (Red, Green, Blue, IR) and photodiode receivers.

- Calculate fluid turbidity, soot concentration, and water emulsion indices in real time.

- Track rate-of-degradation trends to predict fluid replacement dates dynamically.

- Trigger maintenance warnings when fluid opacity or contamination crosses critical safety limits.

3. Key Technical Challenges

- Fluid Viscosity & Bubbles: Filtering out false optical drops caused by transient entrained air bubbles in moving fluid lines.

- Optical Window Staining: Compensating for gradual residue buildup on optical glass sensor windows over time.

- Multi-Fluid Calibration: Adjusting baseline optical absorption profiles across different oil grades and synthetic coolants.

4. Technical Requirements & Deliverables

- Hardware: ESP32 / Arduino Nano, TSL2591 High-Dynamic-Range Light Sensor / TSL230R, Multi-wavelength LED cluster, Transparent inline flow cell.

- DSP Framework: Ratio-metric optical absorption modeling, baseline drift compensation algorithms, and moving-average filters.

- Maintenance UI: Digital health score display (0--100%), estimated remaining fluid life indicator, and alert log output.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Particulate/Soot Correlation | R2≥0.90 correlation with laboratory ISO oil cleanliness codes. |
| Water Ingress Detection | Detects water contamination down to ≥0.5% emulsion concentration. |
| Inline Sensing Latency | Fluid status updated continuously every <2 seconds. |

PS-37: Edge AI Computer Vision Smart Wildlife Crossing & Intrusion Warning

Domain: Environmental Conservation, Highway Safety, & Forestry

Primary Goal: Detect wild animals approaching road corridors or farm perimeters at night using thermal/infrared camera feeds and trigger localized deterrents or motorist warnings.

1. Business Context & Problem Description

Wildlife-vehicle collisions along rural highways cause human casualties, severe vehicle damage, and animal deaths. Furthermore, wild animals intruding into agricultural fields destroy valuable crops overnight. Traditional fencing is expensive and restricts natural migratory paths. An edge vision warning system detects animal approaches and triggers warning lights for drivers or humane acoustic deterrents for crops.

2. Core Objectives

- Capture roadside/perimeter imagery using low-cost Infrared (IR) night-vision or low-resolution thermal cameras.

- Detect and classify wild animals (e.g., deer, wild boars, elephants, cattle) approaching road edges or farm perimeters.

- Activate roadside LED flashing hazard signs to warn approaching motorists in real time.

- Trigger directional ultrasonic frequency bursts to humanely deter crop-raiding animals from entering fields.

3. Key Technical Challenges

- Low-Resolution Night Imagery: Detecting moving animal shapes in grainy IR or low-resolution thermal frames (32×24 or 160×120).

- False Triggers: Rejecting non-target movements such as wind-blown tree branches or small rodents.

- Solar Battery Operations: Operating continuous night surveillance on small solar-recharged power packs.

4. Technical Requirements & Deliverables

- Hardware: ESP32-CAM (IR-cut removed) + IR Illuminator / Raspberry Pi + MLX90640 Thermal Sensor, High-Power Flashing LED Relays, Piezo Ultrasonic Speaker.

- Vision Model: Quantized lightweight object classifier or motion-blob shape analysis model running locally on edge hardware.

- Forestry Dashboard: Wireless event logger transmitting GPS location, timestamped intrusion snapshots, and animal classification counts.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Night Detection Recall | ≥88% detection of large wildlife within a 15-meter sensor perimeter. |
| Driver Alert Speed | Roadside warning signs activated within <500 ms of perimeter breach. |
| Solar Battery Autonomy | Standby operation for ≥3 consecutive rainy days without solar input. |

PS-38: Smart Electrical Sub-Station Thermal Arc Flash & Temperature Mapper

Domain: Industrial Safety, Power Distribution, & Facility Management

Primary Goal: Map surface thermal distribution across high-power busbars and connection lugs continuously using non-contact infrared array sensors to prevent arc flashes.

1. Business Context & Problem Description

Electrical arc flashes in high-power distribution panels occur when loose mechanical busbar joints or degraded contacts overheat, ionizing surrounding air and creating explosive short circuits. Manual thermography inspections performed once a year miss rapidly deteriorating connection joints. Continuous thermal array monitoring provides non-contact, real-time heat mapping inside electrical enclosures.

2. Core Objectives

- Continuous thermal imaging of internal distribution busbars and terminal connections using non-contact IR matrix sensors.

- Interpolate low-resolution thermal data to render continuous 2D temperature heatmaps of the breaker panel interior.

- Track individual terminal connection temperatures against dynamic ambient baseline values.

- Trigger emergency circuit breaker isolation relays before connections reach critical flashover temperatures (>90∘C).

3. Key Technical Challenges

- Emissivity Variances: Adjusting surface temperature calculations across shiny copper busbars, dull steel bolts, and plastic wire insulation.

- Enclosure Space Limits: Wide-angle thermal mapping inside tightly cramped distribution boxes without blind spots.

- Thermal Transient Rejection: Distinguishing normal, temporary current load heat spikes from dangerous contact resistance heating.

4. Technical Requirements & Deliverables

- Hardware: ESP32-S3, MLX90640 (32×24) / AMG8833 Thermal Array Sensor, 12V Contactor Relay, Local Piezo Alarm.

- Software Engine: Dynamic thermal frame interpolation (Bicubic/RBF), temperature anomaly boundary extraction, and historical drift analysis.

- Safety Dashboard: Local screen and web interface rendering color-coded thermal maps with localized connection temperature tags.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Hotspot Precision | Identifies connection overheating within ±1.5∘C accuracy. |
| Arc Flash Prevention SLA | Emergency cutoff triggered within <300 ms of reaching critical thermal limits. |
| Deployment Hardware Cost | Total sensing hardware cost <$45 per distribution panel. |

PS-39: Computer Vision Automated Micro-Solder Joint Defect Inspector

Domain: Electronics Manufacturing, PCB Assembly, & Quality Control

Primary Goal: Inspect micro-solder joints on surface-mount technology (SMT) printed circuit boards using a USB microscope camera and edge computer vision.

1. Business Context & Problem Description

Defective solder joints (e.g., solder bridges, tombstoning, cold joints, insufficient solder) on SMT Printed Circuit Boards (PCBs) cause costly product failures during field operation. Commercial Automated Optical Inspection (AOI) machines cost tens of thousands of dollars, placing them out of reach for small electronics manufacturers and prototyping labs. A desktop camera inspector provides fast, automated solder defect verification at a fraction of the cost.

2. Core Objectives

- Capture high-magnification images of PCB solder pads using a low-cost USB digital microscope camera.

- Segment individual component pads and solder joints using computer vision techniques.

- Classify solder joint quality into specific defect categories (e.g., Good, Solder Bridge, Cold Joint, Missing Component, Tombstone).

- Highlight defect locations visually on a digital PCB placement overlay for operator rework.

3. Key Technical Challenges

- Specular Reflection Glare: Managing extreme light reflections off metallic, curved solder surfaces under bright LED lighting.

- Micro-Scale Precision: Detecting minute solder bridges measuring down to fractions of a millimeter (<0.2 mm).

- Varied Component Geometry: Adapting to diverse component packages (0603, 0805, QFN, SOIC) without requiring re-training.

4. Technical Requirements & Deliverables

- Hardware: Raspberry Pi 4 / PC Workstation, Generic 500× USB Digital Microscope Camera, Ring LED Light Attachment.

- Vision Model: Fine-tuned lightweight object detection / classification network (e.g., YOLO-Nano / MobileNetV3) trained on PCB defect datasets.

- Rework UI: Desktop GUI displaying zoom overlays, color-coded bounding boxes around defective joints, and automated inspection logs.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Solder Defect Recall | ≥95% detection rate across bridging, cold joint, and missing component faults. |
| Inspection Speed | Single component pad evaluation completed within <100 ms. |
| False Inspection Rate | <3% false positive flags on good quality solder joints. |

PS-40: IoT Smart Soil Salinity & Nutrient Leaching Monitor

Domain: Precision Agriculture, Soil Health, & Land Management

Primary Goal: Track soil electrical conductivity (EC), salt accumulation, and fertilizer leaching in greenhouse crops using low-cost multi-depth electrodes.

1. Business Context & Problem Description

Over-fertilization leads to soil salinization, root burn, and chemical leaching into local groundwater, ruining agricultural land and wasting expensive fertilizers. Traditional soil testing requires sending physical samples to laboratories, taking days or weeks for results. Continuous soil Electrical Conductivity (EC) sensing at multi-depth root zones allows farmers to optimize fertilizer application in real time.

2. Core Objectives

- Measure soil Electrical Conductivity (EC), Volumetric Water Content (VWC), and soil temperature at root level continuously.

- Calculate soil salinity indices and track fertilizer salt movement through root zones following irrigation.

- Trigger automated freshwater flushing cycles when soil EC levels exceed crop salt tolerance limits.

- Display real-time soil health matrices and fertilizer retention reports on a mobile app.

3. Key Technical Challenges

- Electrode Polarization: Eliminating DC polarization errors on low-cost stainless steel probe pins during continuous EC measurements.

- Temperature Compensation: Adjusting raw electrical conductivity values to standard 25∘C EC equivalents across changing soil temperatures.

- Soil Matrix Variance: Calibrating EC conversion equations across sand, silt, and potting mix substrates.

4. Technical Requirements & Deliverables

- Hardware: ESP32, Custom AC-Excited Stainless Steel Soil Probes / Low-Cost Analog EC Probe, DS18B20 Temp Sensor, 5V Water Valve Relay.

- Firmware: AC signal generation for anti-polarization measurement, temperature compensation algorithms, and leaching alert triggers.

- Agronomy UI: Mobile interface displaying depth-wise EC trends, soil salinity safety status, and fertigation schedules.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Soil EC Measurement Accuracy | ±5% agreement with laboratory conductivity meter benchmarks. |
| Fertilizer Usage Efficiency | ≥20% reduction in total fertilizer application through real-time EC feedback. |
| Probe Longevity | Stainless steel probes operate without corrosion degradation for ≥6 months. |

PS-41: Edge Computer Vision Smart Conveyor Belt Wear & Rip Detector

Domain: Mining, Heavy Logistics, & Bulk Material Handling

Primary Goal: Detect longitudinal rips, edge fraying, and surface tears on heavy-duty conveyor belts in real time using a line-scan/high-speed USB webcam and edge vision models.

1. Business Context & Problem Description

Industrial conveyor belts transporting coal, ore, or aggregate materials are prone to catastrophic longitudinal rips when sharp debris punctures the belt and gets trapped in the frame. A single undetected rip can slice several kilometers of expensive rubber belt within minutes, causing weeks of plant downtime. An overhead high-speed camera inspecting belt surface integrity detects tears instantly to halt the drive motor.

2. Core Objectives

- Capture continuous high-speed images of moving conveyor belt surfaces using overhead camera feeds.

- Segment belt surface defects (e.g., longitudinal rips, severe cracks, steel cable exposures, edge fraying).

- Compute belt structural health indices and track tear expansion rates over time.

- Trigger immediate emergency motor shutdown relays upon detecting severe longitudinal belt punctures.

3. Key Technical Challenges

- Extreme Motion Speeds: Processing sharp surface frames at belt speeds up to 3--5 meters/second.

- Dust and Dirt Accumulation: Distinguishing structural belt tears from surface dirt streaks, water marks, or carried material residue.

- Sub-Second Motor Trip SLA: Executing image processing and tripping safety contactors in milliseconds to minimize tear length.

4. Technical Requirements & Deliverables

- Hardware: Raspberry Pi 4 / PC Workstation, High-Speed USB 3.0 / RTSP Camera, High-Intensity LED Strip Light, Heavy-Duty Emergency Relay.

- Vision Pipeline: Edge-detection algorithms, line-scan image stitching, and lightweight convolutional tear-detection neural networks.

- Plant Control Dashboard: System GUI displaying live belt surface feeds, defect event logs, and emergency shutdown triggers.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Longitudinal Rip Recall | 100% detection rate for continuous belt punctures ≥10 cm in length. |
| Motor Trip Latency | Emergency shutdown signal generated within <200 ms of rip detection. |
| False Stop Rate | <1 unneeded emergency motor stop per 1,000 operating hours. |

PS-42: IoT Acoustic Transformer Smart Pipe Leak Locator Vector

Domain: Municipal Utilities, Facilities, & Water Conservation

Primary Goal: Locate pinhole water leaks in underground or in-wall plumbing lines by measuring cross-correlation timing of acoustic leak sounds between two strap-on piezo sensors.

1. Business Context & Problem Description

Pressurized water pipes develop small pinhole leaks that remain hidden inside walls or underground for months, wasting huge volumes of water and eroding building foundations. Locating exact leak coordinates traditionally requires destructive wall tearing. Clamping two synchronized acoustic vibration sensors onto exposed pipe ends uses cross-correlation time delays to pinpoint the exact leak distance down the pipe.

2. Core Objectives

- Sample acoustic leak noise signatures simultaneously from two sensors clamped at different exposed points on a pipe line.

- Synchronize sensor sample clocks between sensing nodes using high-precision timers or local microsecond sync.

- Compute the Cross-Correlation Function (CCF) of acoustic signals on edge hardware to calculate wave arrival time delays (Δt).

- Estimate exact leak location distance (X) relative to sensor positions based on pipe material sound velocity constants.

3. Key Technical Challenges

- Microsecond Time Synchronization: Maintaining sub-millisecond clock sync between two independent sensor sampling nodes.

- Variable Acoustic Velocity: Accounting for sound speed differences across steel, copper, PVC, and composite pipe materials.

- Background Noise Rejection: Filtering out ambient water usage noises, pump hums, and footsteps from subtle leak hiss sounds.

4. Technical Requirements & Deliverables

- Hardware: 2x ESP32 Nodes, High-Sensitivity Piezo Contact Microphones, MAX9814 Amplifiers, Pipe Clamps.

- DSP Framework: Dual-channel FFT cross-correlation engine, bandpass acoustic filtering (500 Hz--3 kHz), and distance estimation logic.

- Plumbing UI Engine: Mobile interface showing pipe layout diagrams, acoustic signal overlays, and pin-pointed leak location meters.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Leak Position Accuracy | Pinpoints pinhole leaks within ±0.3 meters on a 20-meter pipe run. |
| Minimum Detectable Leak | Detects pinhole leaks down to ≥0.5 liters/minute under standard pressure. |
| Cross-Correlation Compute | Time-delay estimation calculated in <2 seconds on ESP32. |

PS-43: Computer Vision Smart Silkworm / Insect Larvae Health Classifier

Domain: Sericulture, Sustainable Protein Farming, & Bio-Tech

Primary Goal: Monitor silkworm or Black Soldier Fly Larvae (BSFL) growth rates, tray density, and disease mortality in breeding trays using an overhead camera and computer vision.

1. Business Context & Problem Description

Sericulture (silk production) and insect protein farming (BSFL) rely on healthy larval rearing inside stacked tray systems. Viral infections (e.g., Grasserie disease) spread rapidly across trays, killing entire batches if infected larvae are not picked out immediately. An automated overhead camera system monitors tray density, tracks larval growth curves, and flags dead or diseased larvae early.

2. Core Objectives

- Capture regular overhead images of insect rearing trays using affordable camera modules.

- Estimate larval biomass, density distribution, and individual growth rates automatically.

- Detect and pinpoint discolored, inactive, or dead larvae (e.g., darkened or swollen specimens).

- Alert farm operators to specific trays requiring immediate manual removal of infected specimens.

3. Key Technical Challenges

- High Larval Density & Overlap: Segmenting hundreds of closely packed, wriggling larvae inside a single tray frame.

- Minor Visual Infection Cues: Detecting early viral discoloration before dead bodies decay and infect adjacent larvae.

- Scale Calibration: Tracking growth size increments measured in fractions of a millimeter day-over-day.

4. Technical Requirements & Deliverables

- Hardware: ESP32-CAM / Raspberry Pi Zero 2W + Camera, Overhead Tray Lighting Rig, Buzzer Alert.

- Vision Pipeline: Lightweight instance segmentation / blob detection model tracking larval counts, surface area coverage, and color anomalies.

- Rearing Dashboard: Mobile interface rendering tray-by-tray health status grids, growth rate curves, and disease alert popups.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Mortality / Disease Recall | ≥92% detection rate of diseased or dead larvae within rearing trays. |
| Biomass Estimation Error | Total larval weight estimation accuracy within ±8% of physical scale weights. |
| Inspection Hardware Cost | Hardware setup cost <$20 per tray rack monitored. |

PS-44: Smart HVAC Duct Airflow Anemometer & Damper Balancer

Domain: Green Buildings, Energy Efficiency, & Facility Management

Primary Goal: Balance airflow across central HVAC duct networks automatically by measuring airflow velocity using low-cost thermal anemometer sensors and adjusting motorized zone dampers.

1. Business Context & Problem Description

Centralized HVAC systems in commercial buildings waste energy due to imbalanced airflow distribution—some rooms receive excessive air while others suffer from poor ventilation. Manual airflow balancing using hot-wire anemometers is labor-intensive and performed only during initial installation. Installing permanent, ultra-low-cost thermal airflow sensors inside duct branches enables continuous, automated airflow balancing.

2. Core Objectives

- Measure air velocity (m/s) and air temperature continuously inside branch ventilation ducts using thermal anemometer circuits.

- Calculate volumetric flow rates (CFM) per duct branch based on cross-sectional area profiles.

- Actuate motorized duct damper servo valves automatically to balance airflow across building zones according to occupancy.

- Flag blocked air filters or duct obstructions when airflow drops despite open dampers.

3. Key Technical Challenges

- Low Air Velocity Sensitivity: Accurately sensing low duct air speeds (0.5--2.0 m/s) using low-cost self-heating thermistor circuits.

- Dust Accumulation on Probes: Maintaining sensor calibration when exposed to airborne dust particles inside HVAC return ducts.

- Multi-Zone Control Stability: Preventing oscillating hunting behaviors where adjusting one damper destabilizes airflow in adjacent ducts.

4. Technical Requirements & Deliverables

- Hardware: ESP32 / Arduino, Custom Dual-Thermistor Hot-Wire Anemometer Circuit / FS3000 Air Velocity Sensor, Servo-Driven Duct Damper Flaps.

- Control Software: Multi-zone PID control loop adjusting damper angles based on target room airflow allocations.

- Facility Portal: Dashboard showing real-time duct velocity heatmaps, zone CFM allocations, and duct blockage alert flags.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Air Velocity Accuracy | ±0.2 m/s measurement agreement with commercial vane anemometers. |
| Zone Airflow Balancing SLA | Rebalances zone airflow within <3 minutes of occupancy changes. |
| HVAC Fan Energy Reduction | Reduces total fan power draw by ≥15% through optimized damper positioning. |

PS-45: Optical CV Smart Printed Circuit Board Component Misplacement Inspector

Domain: Electronics Manufacturing, SMT Assembly, & Quality Control

Primary Goal: Verify surface-mount electronic component placement, orientation, and pin 1 polarity alignment on un-soldered PCBs post-pick-and-place using a camera and edge vision.

1. Business Context & Problem Description

In surface-mount technology (SMT) assembly, automated pick-and-place machines occasionally place components out of alignment, rotated incorrectly (90∘/180∘ orientation errors), or with reverse diode/capacitor polarity. If un-soldered PCBs pass into the reflow oven with misplaced components, fixing them requires expensive manual desoldering or discarding the entire board. A post-placement optical camera catches misalignments before reflow soldering.

2. Core Objectives

- Capture high-resolution overhead images of populated, un-soldered PCBs prior to reflow oven entry.

- Compare placed component locations, angles, and polarity dots against target PCB CAD centroids.

- Flag missing parts, component rotation errors, lateral pin shifts, and flipped IC orientations.

- Sound an immediate operator alarm and pause the conveyor feed upon detecting placement faults.

3. Key Technical Challenges

- Un-Soldered Component Displacement: Inspecting components without physical contact or air turbulence that could bump un-soldered parts out of place.

- Polarity Dot & Text Identification: Reading tiny etched IC orientation markers and diode polarity bands under varying lighting glare.

- High-Speed Inspection SLA: Inspecting 50--200 components per board within a few seconds to match pick-and-place speeds.

4. Technical Requirements & Deliverables

- Hardware: Raspberry Pi 4 / PC Workstation, High-Resolution USB Camera module, Enclosed LED Ring Diffuser Light, Conveyor Pause Relay.

- Vision Engine: OpenCV template matching, feature keypoint alignment (ORB/SIFT), and OCR polarity verification logic.

- SMT Quality UI: Operator screen showing PCB layout overlays, highlighted red error boxes around misplaced components, and inspection logs.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Placement Defect Recall | ≥98% detection rate for rotated, misplaced, or missing SMT components. |
| Inspection Cycle Time | Complete PCB scan (>100 components) evaluated within <3 seconds. |
| False Rejection Rate | <2% false alarms on correctly placed components. |

PS-46: Smart Piezoelectric Energy Harvesting Floor Mat Occupancy Counter

Domain: Smart Buildings, Energy Harvesting, & Privacy-Preserving Sensing

Primary Goal: Count human footfalls and estimate directional foot traffic through building doorways using self-powered piezoelectric floor mats without external power supplies.

1. Business Context & Problem Description

Retail stores and smart buildings track occupant foot traffic to optimize staffing and HVAC usage. Optical cameras raise privacy concerns, while battery-powered door sensors require frequent battery changes across large facilities. A floor mat embedded with low-cost piezoelectric elements generates its own electrical energy from human steps, powering a low-power wireless micro-transmitter to send footfall events without batteries or external wiring.

2. Core Objectives

- Harvest electrical energy from human footstep pressure using piezoelectric ceramic discs embedded inside a thin floor mat.

- Rectify and store harvested footstep energy in a small supercapacitor.

- Wake an ultra-low-power wireless micro-controller node using the harvested footstep energy burst.

- Transmit a wireless packet counting the footfall event to a central building receiver before returning to zero-power sleep.

3. Key Technical Challenges

- Energy Harvesting Efficiency: Harvesting sufficient micro-Joules (uJ) from a single step to power a complete wireless transmission cycle.

- Ultra-Fast Microcontroller Boot: Executing code, reading step directional sensors, and transmitting data in under 10 milliseconds.

- Mechanical Mat Durability: Protecting ceramic piezo discs from mechanical fracturing under heavy, repeated foot traffic.

4. Technical Requirements & Deliverables

- Hardware: Piezoelectric Ceramic Discs (e.g., 35 mm PZT), Bridge Rectifier Diodes, Energy Harvesting Management IC (e.g., LTC3588), Supercapacitor, ESP32-C3 / nRF52840 (Ultra-low power mode).

- Firmware: Fast-boot bare-metal code executing instant RF packet transmission upon power-up.

- Foot Traffic Dashboard: Wireless receiver node and web application logging hourly footfall counts, doorway directionality, and peak traffic hours.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Self-Powered Transmission Rate | 100% wireless transmission success from steps ≥40 kg body weight. |
| External Power Requirement | 0 Watts (100% self-powered operation via harvested footfall energy). |
| Mat Mechanical Lifespan | Mat survives ≥500,000 footfall cycles without mechanical degradation. |

PS-47: Computer Vision Smart Silo Material Level & Bridging Inspector

Domain: Industrial Logistics, Material Handling, & Bulk Storage

Primary Goal: Measure bulk material fill levels (powders, grains, plastic pellets) and detect internal "bridging" or "rat-holing" blockages inside silos using an internal optical camera and edge AI.

1. Business Context & Problem Description

Bulk materials (e.g., flour, cement, plastic pellets, sawdust) stored in industrial silos often suffer from internal structural blockages known as "bridging" (an empty arch forming over the outlet) or "rat-holing" (a narrow vertical hole emptying while material clings to side walls). Traditional single-point ultrasonic depth sensors report false fill levels during bridging events. An internal dome camera paired with edge vision tracks true 3D material volume and flags dangerous wall blockages.

2. Core Objectives

- Capture wide-angle top-down images of internal silo material surfaces using a dust-protected camera setup.

- Calculate true multi-point material volume and remaining percentage capacity using computer vision.

- Detect structural flow blockages (e.g., wall cling, rat-hole funnels, material bridging arches).

- Trigger automated bin-vibrator motors to break up detected material bridges before line shutdowns occur.

3. Key Technical Challenges

- Airborne Dust & Lens Coating: Preventing fine dust particles (e.g., flour, cement) from coating internal camera lenses and illumination LEDs.

- Low-Contrast Texture Analysis: Segmenting uniform-color bulk powders (e.g., white plastic pellets) under directional lighting.

- 3D Surface Reconstruction: Estimating non-planar bulk material volumes from 2D top-down camera views.

4. Technical Requirements & Deliverables

- Hardware: ESP32-CAM / Raspberry Pi with Wide-Angle Lens, Air-Pulsed Self-Cleaning Glass Enclosure, LED Ring, 12V Relay driving Bin Vibrator.

- Vision Pipeline: Texture gradient analysis, shadows-from-shape depth estimation, and structural blockage pattern recognition.

- Silo Operations Portal: Web interface displaying 3D material topography models, true volume percentages, and automated vibrator event logs.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| True Volume Error | Fill volume estimation accuracy within ±6% for non-uniform bulk surfaces. |
| Bridging Detection Recall | ≥94% detection of rat-hole funnels and material wall-adhesion arches. |
| Lens Self-Cleaning SLA | Air-purge mechanism maintains clear optical view across ≥30 days of continuous dusting. |

PS-48: Smart Inductive Sensor Metal Wear Debris Counter for Gearboxes

Domain: Heavy Machinery, Predictive Maintenance, & Mining Equipment

Primary Goal: Count and size metallic wear debris particles floating in heavy machinery gearbox oil lines in real time using a low-cost inductive coil sensor setup.

1. Business Context & Problem Description

Heavy-duty industrial gearboxes (e.g., in wind turbines, mining crushers, locomotives) shed tiny metallic wear flakes into their lubricating oil as gears and bearings degrade. Waiting for routine oil samples to be sent to offline laboratories delays fault detection until major gear teeth break. An inline inductive coil sensor wrapped around the oil bypass tube counts and sizes metallic wear particles in real time, catching component pitting early.

2. Core Objectives

- Pass gearbox lubricating oil through a non-invasive tubing loop wrapped with micro-inductive coils.

- Sense inductance changes (ΔL) caused by metallic wear particles passing through the coil magnetic field.

- Distinguish ferrous metal particles (iron, steel) from non-ferrous wear particles (copper, brass) based on phase shifts.

- Count particle frequency and categorize sizes (e.g., <50μm,50--100μm,>100μm) to trigger gearbox overhaul warnings.

3. Key Technical Challenges

- Minute Inductance Shifts: Detecting micro-Henry (μH) inductance perturbations caused by tiny microscopic metal flakes.

- High-Speed Signal Sampling: Sampling high-frequency LC oscillator circuits fast enough to count individual fast-moving oil debris particles.

- Vibration Noise Rejection: Shielding sensing coils from external electromagnetic interference and mechanical machine vibration.

4. Technical Requirements & Deliverables

- Hardware: ESP32 / Teensy 4.0, Custom LC Resonant Oscillator Sensing Coil, LDC1612 / LDC1101 Inductive Sensing IC, OLED Display.

- DSP Framework: High-speed frequency shift detection, pulse peak threshold counting, and particle size classification algorithms.

- Gearbox Health Portal: Real-time UI displaying cumulative metallic particle counts, size distribution histograms, and health alert gauges.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Particle Size Detection Limit | Detects ferrous wear particles down to ≥40 μm particle diameter. |
| Ferrous/Non-Ferrous Differentiation | ≥90% accuracy in distinguishing iron/steel from brass/copper wear flakes. |
| Inline Flow Rate Tolerance | Operates reliably at oil flow velocities up to 1.0 meters/second. |

PS-49: Edge Computer Vision Smart Solar Tracker & Cloud Shadow Predictor

Domain: Renewable Energy, Solar Power, & Precision Controls

Primary Goal: Predict incoming cloud shadow coverage and optimize solar panel tilt angles dynamically using an upward-facing fisheye camera and edge vision algorithms.

1. Business Context & Problem Description

Traditional solar tracking systems calculate panel angles based purely on astronomical sun position algorithms, assuming clear skies. When heavy clouds pass overhead, diffuse sunlight becomes dominant, making flat panel angles more efficient than pointing directly at the obscured sun. Furthermore, sudden cloud shadows cause sharp power drops across solar farms. An upward-facing fisheye camera tracks cloud velocity to predict shadows and re-orient panel angles dynamically.

2. Core Objectives

- Capture continuous sky condition images using an upward-facing wide-angle/fisheye camera module.

- Segment cloud boundaries, identify cloud types, and calculate cloud movement velocity vectors across the sky.

- Predict impending cloud shadow arrival times on specific solar panel strings (10--60 seconds in advance).

- Adjust motorized solar panel array tilt angles dynamically to maximize power output under both direct and diffuse light.

3. Key Technical Challenges

- Direct Sun Glare Overexposure: Processing sky imagery without sensor saturation caused by direct sunlight.

- Cloud Motion Prediction: Tracking non-rigid, fast-changing cloud formations and estimating accurate velocity vectors.

- Net Power Optimization: Ensuring the extra energy gained by dynamic tracking exceeds the power consumed by panel tilt motors.

4. Technical Requirements & Deliverables

- Hardware: ESP32-CAM / Raspberry Pi with 180∘ Fisheye Lens, Optical Density Sun Filter, Dual-Axis Stepper Motor Drivers.

- Vision Pipeline: Sky-condition thresholding, optical flow motion vector calculation, and diffuse-vs-direct irradiance prediction model.

- Solar Controller Portal: Real-time web UI displaying live sky vectors, predicted shadow arrival countdowns, and generated power logs.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Shadow Arrival Prediction Time | Predicts cloud shadow events 10--45 seconds prior to panel arrival. |
| Solar Yield Improvement | ≥5--8% net energy gain compared to standard astronomical sun trackers on cloudy days. |
| Tracking Hardware Cost | Sensing and compute hardware setup cost <$35 per array controller. |

PS-50: Smart Acoustic Door Slam & Window Intrusion Detector

Domain: Home Security, Building Automation, & Privacy-Preserving Sensing

Primary Goal: Detect glass breaking, door forcing, and structural intrusion events using a single low-cost acoustic sensor node and edge waveform pattern matching without cloud cameras.

1. Business Context & Problem Description

Home intrusion detection typically relies on physical door contact sensors on every window, motion detectors in every room, or invasive interior video cameras that compromise privacy. High-frequency acoustic vibrations caused by breaking glass, door forcing, or lock picking propagate through air and building frames. A single, privacy-focused acoustic edge node classifies structural intrusion sounds in real time while ignoring normal household noise.

2. Core Objectives

- Capture ambient indoor acoustic signals continuously using a low-cost MEMS microphone.

- Analyze acoustic frequency spectra and temporal envelope profiles on edge hardware.

- Classify acoustic events into normal home sounds (voices, TV, door closing, dropped objects) vs. intrusion events (shattering glass, wood splitting, heavy impact door forcing).

- Trigger local siren alarms and transmit instant intrusion alerts to home owners within milliseconds.

3. Key Technical Challenges

- Glass Break Waveform Matching: Recognizing distinct dual-frequency glass shattering signatures (initial flex pop followed by high-frequency glass tinkling).

- Zero False Alarms: Eliminating false alarms caused by TV action movies, barking dogs, or kitchen dish clatter.

- 100% Offline Edge Operation: Running complete audio feature extraction and inference locally to guarantee absolute privacy and zero internet dependency.

4. Technical Requirements & Deliverables

- Hardware: ESP32-S3 / Raspberry Pi Pico W, INMP441 I2S Digital MEMS Microphone module, Piezo Siren Relay, Indicator LEDs.

- Edge Audio Engine: On-device spectrogram extraction, zero-crossing rate analysis, and lightweight TinyML acoustic classifier model.

- Security UI Engine: Standalone device configuration web portal, sensitivity tuning wizard, and event snapshot log history.

5. Evaluation Metrics & Success Criteria

| Metric | Description / Target |
| --- | --- |
| Glass Break & Intrusion Recall | ≥96% detection accuracy for glass shattering and forced structural entry. |
| False Positive Rate | <1% false alarms triggered by ambient household noises or television playback. |
| Inference & Alert SLA | Siren alarm triggered within <100 ms of acoustic intrusion event. |